Blantons FPGA Register Definition

Version 0.3 (draft)

7 April, 2026

Hewlett-Packard Enterprise

Authored by:

Peter, Liew

HPE Company Confidential

# Table of Contents

Contents

1	Table of Contents	2

2	Revision history	7

3	Document Abbreviation	8

4	MSI Interrupts	9

4.1	Interrupt registers	9

4.2	MSI Generation	10

5	LED Control bits	10

6	Common Register Description	11

6.1	FPGA_REV_ID_REG	11

6.2	RDND_MGMT_REG	11

6.3	FUNC_CAP_0_REG	11

6.4	FUNC_CAP_1_REG	12

6.5	FUNC_CAP_2_REG	12

6.6	FUNC_CAP_3_REG	12

6.7	FUNC_CAP_4_REG	13

6.8	FUNC_CAP_5_REG	13

6.9	FUNC_CAP_6_REG	14

6.10	FUNC_CAP_7_REG	14

6.11	FUNC_CAP_8_REG	14

6.12	MBOX_n_DATA_REG	15

6.13	MBOX_STAT_REG	15

6.14	MBOX_STAT_PLRTY_REG	16

6.15	MBOX_STAT_EDG_LVL_REG	16

6.16	MBOX_STAT_INT_EN_REG	17

6.17	MBOX_STAT_INT_STAT_REG	17

6.18	MBOX_STAT_INT_CLR_REG	18

6.19	SCRTCH_n_REG	18

6.20	SEM_REG	18

6.21	SPI_CMD_STAT_REG	19

6.22	SPI_ADD_REG	20

6.23	SPI_WR_DATA_REG	21

6.24	SPI_RD_DATA_REG	21

6.25	SPI_RST_REG	21

6.26	I2C_PRSCL_LO_REG	22

6.27	I2C_PRSCL_HI_REG	22

6.28	I2C_CTRL_REG	22

6.29	I2C_TX_REG	23

6.30	I2C_RX_REG	23

6.31	I2C_CMD_REG	24

6.32	I2C_STAT_REG	24

6.33	I2C_MUX_SEL_REG	25

6.34	I2C_RST_REG	25

6.35	LED_SYNC_REG	26

6.36	LED_CONFIG_REG	26

6.37	FPGA_RECONFIG_CTRL_REG	27

6.38	FPGA_RECONFIG_STAT_REG	28

7	CB Registers	29

Function 0	29

7.1	FPGA_REV_ID_REG	29

7.2	FUNC_CAP_0_REG	29

7.3	FPGA_RECONFIG_CTRL_REG	29

7.4	FPGA_RECONFIG_STAT_REG	29

7.5	SCRTCH_n_REG	29

7.6	DEV_MSI_0_PEND_REG (TBD)	29

7.7	DEV_MSI_0_INT_EN_REG (TBD)	29

7.8	PWR_CTRL_REG	29

7.9	PWR_STAT_REG	31

7.10	RS232_SPD_SNS_REG	31

7.11	RS232_CLR_SPD_SNS_REG	31

7.12	TPM_STAT_REG	32

7.13	PMC_CTRL_REG	32

7.14	PMC_STAT_REG	32

7.15	ICB_CTRL_REG	33

7.16	ICB_STAT_REG	34

7.17	SWB_CTRL_REG	35

7.18	SWB_STAT_REG	36

7.19	BMC_CTRL_REG	37

7.20	BMC_STAT_REG	38

7.21	THRM_STAT_REG	38

7.22	USB_CTRL_REG	39

7.23	USB_STAT_REG	39

7.24	BTN_STAT_REG	39

7.25	LEAK_STAT_REG	40

7.26	NFC_CTRL_REG	40

7.27	NFC_STAT_REG	40

7.28	INTRD_STAT_REG	41

7.29	BTRY_STAT_REG	41

7.30	EN_ACCSS_PRTCTD_REG	41

7.31	UART_MUX_REG	42

7.32	I2C_MUX_RST_REG	43

7.33	CPU_INTR_CTRL_REG	43

7.34	BMC_JTAG_CTRL_REG	43

7.35	SWB_JTAG_CTRL_REG	44

7.36	FLSH_CTRL_REG	44

7.37	LED_SYNC_REG	45

7.38	LED_CONFIG_REG	45

7.39	LED_CTRL_0_REG	45

7.40	LED_CTRL_1_REG	45

7.41	LED_CTRL_2_REG	46

7.42	LED_CTRL_3_REG	46

7.43	LED_CTRL_4_REG	46

7.44	LED_CTRL_5_REG	47

7.45	WDG_CTRL_REG	47

7.46	WDG_STAT_REG	48

7.47	WDG_RST_REG	48

7.48	TND_CTRL_REG	49

7.49	TND_STAT_REG	49

7.50	TND_PWR_CTRL_REG	49

7.51	TND_PWR_STAT_REG	49

7.52	TND_CLK_CTRL_REG	50

7.53	TND_PMC_GPIO_CTRL_REG	50

7.54	TND_PMC_GPIO_STAT_REG	51

7.55	TND_PMC_GPIO_DIR_CTRL_REG	51

7.56	TND_BMC_GPIO_CTRL_REG	52

7.57	TND_BMC_GPIO_STAT_REG	52

7.58	TND_BMC_GPIO_DIR_CTRL_REG	53

7.59	TND_SWB_GPIO_CTRL_REG	53

7.60	TND_SWB_GPIO_STAT_REG	53

7.61	TND_SWB_GPIO_DIR_CTRL_REG	54

7.62	TND_ICB_GPIO_CTRL_REG	54

7.63	TND_ICB_GPIO_STAT_REG	54

7.64	TND_ICB_GPIO_DIR_CTRL_REG	55

7.65	RST_CAUSE_REG	55

7.66	RST_CAUSE_SWB_REG	56

Function 1	56

7.67	FPGA_REV_ID_REG	56

7.68	FUNC_CAP_0_REG	56

7.69	SCRTCH_n_REG	56

7.70	DEV_MSI_0_PEND_REG (TBD)	56

7.71	DEV_MSI_0_INT_EN_REG (TBD)	57

7.72	FAN_CTRL_REG	57

7.73	FAN_STAT_REG	58

7.74	RTM_CTRL_REG	59

7.75	RTM_STAT_REG	59

7.76	ETH_CTRLR_CTRL_REG	59

7.77	ETH_CTRLR_STAT_REG	60

7.78	USRPRT_PWR_CTRL_REG	60

7.79	USRPRT_PWR_STAT_REG	61

7.80	USRPRT_CTRL_0_REG	61

7.81	USRPRT_STAT_0_REG	61

7.82	USRPRT_CTRL_1_REG	62

7.83	USRPRT_STAT_1_REG	62

7.84	PHY_CTRL_REG	62

Function 2	63

7.85	FPGA_REV_ID_REG	63

7.86	FUNC_CAP_0_REG	63

7.87	SCRTCH_n_REG	63

7.88	DEV_MSI_0_PEND_REG (TBD)	63

7.89	DEV_MSI_0_INT_EN_REG (TBD)	63

7.90	SPI_CMD_STAT_REG	64

7.91	SPI_ADD_REG	64

7.92	SPI_WR_DATA_REG	64

7.93	SPI_RD_DATA_REG	64

7.94	SPI_RST_REG	64

7.95	GEN_SPI_RD_DATA_REG	64

7.96	GEN_SPI_WR_DATA_REG	64

7.97	GEN_SPI_STAT	64

7.98	GEN_SPI_CTRL_REG	65

7.99	GEN_SPI_CFG_CTRL_REG	66

7.100	GEN_SPI_SLV_SEL_REG	67

7.101	MDIO_STAT_CMD_REG	68

7.102	MDIO_CTRL_REG	69

7.103	MDIO_STK_CTRL_REG	70

7.104	MDIO_STK_ADDR_CONF_REG	70

7.105	MDIO_STK_STAT_REG	71

7.106	MDIO_STK_RAM_DATA_REG	71

7.107	MDIO_STK_RAM_ADDR_REG	72

7.108	MDIO_STAT_INT_REG	72

Function 3	73

7.109	FPGA_REV_ID_REG	73

7.110	FUNC_CAP_0_REG	73

7.111	SCRTCH_n_REG	73

7.112	DEV_MSI_0_PEND_REG (TBD)	73

7.113	DEV_MSI_0_INT_EN_REG (TBD)	73

7.114	I2C_PRSCL_LO_REG	74

7.115	I2C_PRSCL_HI_REG	74

7.116	I2C_CTRL_REG	74

7.117	I2C_TX_REG	74

7.118	I2C_RX_REG	74

7.119	I2C_CMD_REG	74

7.120	I2C_STAT_REG	74

7.121	I2C_MUX_SEL_REG	74

7.122	I2C_RST_REG	74

8	SWB Registers	75

8.1	FPGA_REV_ID_REG	75

8.2	FPGA_RECONFIG_CTRL_REG	75

8.3	FPGA_RECONFIG_STAT_REG	75

8.4	SCRTCH_n_REG	75

8.5	SWB_INFO_REG	75

8.6	PWR_STAT_REG	75

8.7	CB_CTRL_REG	75

8.8	CB_STAT_REG	76

8.9	ASIC_CTRL_REG	77

8.10	ASIC_STAT_REG	78

8.11	RTM_CTRL_REG	78

8.12	THRM_STAT_REG	78

8.13	EN_ACCSS_PRTCTD_REG	79

8.14	I2C_MUX_RST_REG	79

8.15	SPI_MUX_REG	80

8.16	FLSH_CTRL_REG	80

8.17	TND_PWR_CTRL_0_REG	81

8.18	TND_PWR_CTRL_1_REG	82

8.19	TND_PWR_STAT_0_REG	83

8.20	TND_PWR_STAT_1_REG	84

8.21	TND_PWR_STAT_2_REG	84

8.22	TND_PWR_STAT_3_REG	86

8.23	TND_CB_GPIO_CTRL_REG	86

8.24	TND_CB_GPIO_STAT_REG	86

8.25	TND_CB_GPIO_DIR_CTRL_REG	86

8.26	RST_CAUSE_REG	87

9	ICB Registers	88

9.1	FPGA_REV_ID_REG	88

9.2	FPGA_RECONFIG_CTRL_REG	88

9.3	FPGA_RECONFIG_STAT_REG	88

9.4	SCRTCH_n_REG	88

9.5	PWR_CTRL_REG	88

9.6	PWR_STAT_REG	88

9.7	FAN_CTRL_REG	89

9.8	FAN_STAT_REG	91

9.9	THRM_STAT_REG	91

9.10	EN_ACCSS_PRTCTD_REG	91

9.11	I2C_MUX_RST_REG	92

9.12	FLSH_CTRL_REG	92

9.13	LED_SYNC_REG	93

9.14	LED_CONFIG_REG	93

9.15	LED_CTRL_REG	93

9.16	TND_CB_GPIO_CTRL_REG	93

9.17	TND_CB_GPIO_STAT_REG	93

9.18	TND_CB_GPIO_DIR_CTRL_REG	94

# Revision history

| Revision No. | Date mm/dd/yy | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 03/20/2026 | Peter | 1st Draft |
| 0.2 | 03/23/2026 | Peter | Reviewed by HW |
| 0.3 | 04/06/2026 | Peter | CB - F0 PWR_CTRL/STAT changes - Move USRPRT power to F1 USRPRT_PWR_CTRL/STAT - Move USB 5V to USB_CTRL_REG - Added separate THRM_SUICIDE bit control for SWB 0/1 - Added power status of SWB0/1 - #SWB<no>_PWROK - USB_STAT_REG – added new detection bit [1] - ICB_CTRL_REG [3:0] – added encoded reset control to ICB - Fix minor typo in F1 registers - F1 USRPRT_PWR_CTRL – Efuse to QSFP are default to disabled - TND_BMC_GPIO_CTRL/STAT/DIR – replace bit [7] with [0] - Updated LED_CTRL_4/5_REG description - Decouple SWB reset signal into RST_CAUSE_SWB_REG (from RST_CAUSE_REG) - TND_PWR_CTRL/STAT_REG – added new power rails control and status bits - F1 RTM_CTL_REG #RTM_RST default change to asserted |

# Document Abbreviation

TBD:		To be determined

W1C:		Write 1 to clear

W0C:		Write 0 to clear

RO:		Read Only

WO:		Write Only

RW:		Read & Write

# MSI Interrupts

## Interrupt registers

- MSI vector number assigned for each interrupt is predefined and not programmable.

- There is a pair of DEV_MSI_PEND_REG and DEV_MSI_INT_EN_REG register for each MSI vector.

- Each device within the function will have a PEND bit in one of the DEV_MSI_PEND_REG registers according to MSI vector number assigned to that device.

- A device will have its PEND bit set when it is requesting service.

- There can be as many as 32 pair of PEND/EN registers (one for each MSI interrupt vector) in a function. The actual number of MSI vectors reserved for each function is currently TBD.

- PEND/EN registers are 32 bit registers but they can be extended to 64 bit if more than 32 interrupt request are assigned to a single MSI (A 64 bit register is composed of two 32-bit registers).

#### DEV_MSI_PEND_REG

There can be as many as 32 DEV_MSI_PEND_REG registers (one for each MSI interrupt vector) per function. Each device within the function will have a PEND bit in one of the PEND registers. A device will have its PEND bit set when it is requesting service.

| #Register Name |  | DEV_MSI_PEND_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #TBD |  | RO |  |  |
| #TBD |  | RO |  |  |

#### DEV_MSI_INT_EN_REG

There can be as many as 32 DEV_MSI_INT_EN_REG registers (one for each MSI interrupt vector) per function. The actual number of MSI vectors reserved for each function or device is currently TBD.

Enables a device to have its interrupt PEND status to trigger a MSI interrupt.

| #Register Name |  | DEV_MSI_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #TBD |  | RW |  |  |
| #TBD |  | RW |  |  |

## MSI Generation

FPGA will trigger MSI interrupt only when (INT_EN [MSI_INDEX] & PEND [MSI_INDEX]) transition from 0 to 1. To avoid missing interrupts SW would make sure that it does not exit the ISR until the condition (INT_EN [MSI_INDEX] & PEND [MSI_INDEX]) is zero. Following examples clarifies how MSI interrupts are generated and serviced by FPGA and SW.

Scenario 1:  Assume at time 0 no interrupt is pending and DEV1-INT is enabled (means its respective bit in DEV_MSI_INT_EN_REG is asserted). At time t1, PEND bit for DEV1-INT transitions from 0 to 1. Since enable bit is already 1, FPGA will detect 01 transition in (INT_EN & PEND) therefore will send MSI to CPU.

Scenario 2:  Assume at time 0, DEV1-INT is pending but disabled. At time t1 SW set the DEV1-INT enable bit to 1. Since PEND bit is already 1, FPGA will detect 01 transition in (INT_EN & PEND) therefore will send MSI to CPU.

Scenario 3:  Assume at time 0 no interrupt is pending and DEV1-INT and DEV2-INT are enabled (means their respective bits in DEV_MSI_INT_EN_REG are asserted). At time t1, PEND bit for DEV1-INT transitions from 0 to 1. Since enable bit is already 1, FPGA will detect 01 transition in (INT_EN & PEND) therefore will send MSI to CPU. Immediately after sending MSI and before SW can service DEV1-INT, at time t1+e, PEND bit for DEV2-INT transitions from 0 to 1. At time t1+e, (INT_EN & PEND) is already 1 and FPGA will not detect any 01 transition so no new MSI will be generated. At the time SW services the first MSI, it will check the PEND register and find out that DEV1-INT and DEV2-INT is pending and will service both interrupt requests before exiting the ISR.

Scenario 4:  Assume at time 0 no interrupt is pending and DEV1-INT and DEV2-INT are enabled (means their respective bits in DEV_MSI_INT_EN_REG are asserted). At time t1, PEND bit for DEV1-INT transitions from 0 to 1. Since enable bit is already 1, FPGA will detect 01 transition in (INT_EN & PEND) therefore will send MSI to CPU. As part of ISR, SW will read PEND register and find out that Dev1-INT is pending. Before SW can fully service the DEV1-INT and clear the respective PEND bit, at time t2, PEND bit for DEV2-INT transitions from 0 to 1. At time t2, (INT_EN & PEND) is already 1 and FPGA will not detect any 01 transition so no new MSI will be generated. After SW cleared DEV1-INT before exiting the ISR, it will again read PEND register and find out that DEV2-INT is pending. SW will continue with servicing DEV2-INT before exiting the ISR.

Scenario 5: at Scenario 4, after SW cleared DEV1-INT (INT_EN & PEND transition from 1 to 0), immediately DEV2-INT happens. Before exiting ISR, SW will read the PEND register and detect the DEV2-INT but at the same time FPGA will detect 01 transition in (INT_EN & PEND) and send MSI. In this scenario SW services DEV2-INT as part of first MSI interrupt, therefore when SW process the second MSI it finds no pending INT because DEV2-INT has already been serviced by first MSI. This situation should be managed by SW.

# LED Control bits

The status of each LED is defined by an 8-bit register as explained below:

Bit [0] – 0 off, 1 on

Bit [1] – 0 orange, 1 green or single color

Bit [2] – 0 not blink, 1 blink

Bit [3] – 0 slow blink, 1 fast blink

Bit [4] – 0 full bright, 1 half bright

Bit [5] – Reserved

Bit [7:6] – This bit is reserved unless stated in description

0 FPGA/SW Ctrl, other values (external control)

All LED register in this document use above codes to define the status of LED.

# Common Register Description

## FPGA_REV_ID_REG

| #Register Name |  | FPGA_REV_ID_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Stores Device-ID & Revision Number for FPGA |  |  |
| POR Value |  | See Description |  |  |
| Field | Bits | Type | Reset | Description |
| #FPGA_ID | 31:16 | RO | See Description | Device Identification, CB = 0x2001 SWB = 0x2101 ICB = 0x2201 |
| #FPGA_REV | 15:0 | RO | 0x0001 | FPGA Code Revision field. Incremented by 1 in each revision. For lab Release, Rev will start with “1” illustrating beta version. e.g. 0x8001 |

## RDND_MGMT_REG

| #Register Name |  | RDND_MGMT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Redundancy Management Registers |  |  |
| POR Value |  | 0xCCCC_CCCC |  |  |
| Field | Bits | Type | Reset | Description |
| #RDND_MGMT_STAT | 31:0 | RW | 0xCCCC_CCCC | Under SW control |

## FUNC_CAP_0_REG

| #Register Name |  | FUNC_CAP_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_3_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 3 |
| #MSI_2_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 2 |
| #MSI_1_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 1 |
| #MSI_0_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 0 |

## FUNC_CAP_1_REG

| #Register Name |  | FUNC_CAP_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_7_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 7 |
| #MSI_6_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 6 |
| #MSI_5_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 5 |
| #MSI_4_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 4 |

## FUNC_CAP_2_REG

| #Register Name |  | FUNC_CAP_2_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_11_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 11 |
| #MSI_10_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 10 |
| #MSI_9_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 9 |
| #MSI_8_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 8 |

## FUNC_CAP_3_REG

| #Register Name |  | FUNC_CAP_3_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_15_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 15 |
| #MSI_14_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 14 |
| #MSI_13_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 13 |
| #MSI_12_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 12 |

## FUNC_CAP_4_REG

| #Register Name |  | FUNC_CAP_4_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_19_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 19 |
| #MSI_18_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 18 |
| #MSI_17_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 17 |
| #MSI_16_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 16 |

## FUNC_CAP_5_REG

| #Register Name |  | FUNC_CAP_5_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_23_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 23 |
| #MSI_22_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 22 |
| #MSI_21_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 21 |
| #MSI_20_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 20 |

## FUNC_CAP_6_REG

| #Register Name |  | FUNC_CAP_6_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_27_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 27 |
| #MSI_26_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 26 |
| #MSI_25_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 25 |
| #MSI_24_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 24 |

## FUNC_CAP_7_REG

| #Register Name |  | FUNC_CAP_7_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined per each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #MSI_31_INT_NUM | 31:24 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 31 |
| #MSI_30_INT_NUM | 23:16 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 30 |
| #MSI_29_INT_NUM | 15:8 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 29 |
| #MSI_28_INT_NUM | 7:0 | RO |  | Illustrates the Number of interrupts that can generate MSI vector 28 |

## FUNC_CAP_8_REG

| #Register Name |  | FUNC_CAP_8_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Defines the capability of PCIe function |  |  |
| POR Value |  | Defined for each function of each FPGA. |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO |  | Reserved |
| #NUM_OF_PRTS | 7:0 | RO |  | Represent the number of I2C ports in function 3. This is inclusive of real I2C ports, Virtual I2C Ports, and reserved I2C ports. This field is not used in other functions and will be always 0. |

## MBOX_n_DATA_REG

Data will be written in this register only if respective stat bit in MBOX_STAT_REG is showing empty stat. Write in this register will set the respective stat bit in MBOX_STAT_REG.

| #Register Name |  | MBOX_n_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  | Mailbox data |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #DATA | 31:0 | RW | 0 |  |

## MBOX_STAT_REG

| #Register Name |  | MBOX_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  | Mailbox status |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_STAT | 3 | W1C | 0 | 1: MBOX_3_DATA_REG is full. 0: MBOX_3_DATA_REG is empty. |
| #MBOX_2_STAT | 2 | W1C | 0 | Same as MBOX_3_STAT |
| #MBOX_1_STAT | 1 | W1C | 0 | Same as MBOX_3_STAT |
| #MBOX_0_STAT | 0 | W1C | 0 | Same as MBOX_3_STAT |

## MBOX_STAT_PLRTY_REG

Defines which level of the MBOX_STAT signal is the condition for triggering the Interrupt.

STAT_PLRTY_REG is defined once in this document. All other STAT_PLRTY_REG registers’ definition will be similar to this and their bits will follow their respective STAT_REG.

| #Register Name |  | MBOX_STAT_PLRTY_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_STAT_PLRTY | 3 | RW | 0 | 1: means MBOX_3_STAT hi (1’b1) is the condition for triggering the interrupt. In case of edge interrupt a stat transition from lo to hi (1’b0 to 1’b1) is the condition for triggering the interrupt. 0: means MBOX_3_STAT lo (1’b0) is the condition for triggering the interrupt. In case of edge interrupt a stat transition from hi to lo (1’b1 to 1’b0) is the condition for triggering the interrupt. |
| #MBOX_2_STAT_PLRTY | 2 | RW | 0 | Same as MBOX_3_STAT_PLRTY |
| #MBOX_1_STAT_PLRTY | 1 | RW | 0 | Same as MBOX_3_STAT_PLRTY |
| #MBOX_0_STAT_PLRTY | 0 | RW | 0 | Same as MBOX_3_STAT_PLRTY |

## MBOX_STAT_EDG_LVL_REG

Defines if the level or the transition of the MBOX_STAT signal is the condition for triggering the Interrupt.

STAT_EDG_LVL_REG is defined once in this document. All other STAT_EDG_LVL_REG registers’ definition will be similar to this and their bits will follow their respective STAT_REG.

| #Register Name |  | MBOX_STAT_EDG_LVL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_STAT_EDG_LVL | 3 | RW | 0 | 1: means the transition (or the edge) of MBOX_3_STAT is the condition for triggering the interrupt. 0: means the level of MBOX_3_STAT is the condition for triggering the interrupt. |
| #MBOX_2_STAT_EDG_LVL | 2 | RW | 0 | Same as MBOX_3_STAT_EDG_LVL |
| #MBOX_1_STAT_EDG_LVL | 1 | RW | 0 | Same as MBOX_3_STAT_EDG_LVL |
| #MBOX_0_STAT_EDG_LVL | 0 | RW | 0 | Same as MBOX_3_STAT_EDG_LVL |

## MBOX_STAT_INT_EN_REG

Enable interrupts for status bits of mailboxes.

STAT_INT_EN_REG is defined once in this document. All other STAT_INT_EN_REG registers’ definition will be similar to this and their bits will follow their respective STAT_REG.

| #Register Name |  | MBOX_STAT_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_STAT_EN | 3 | RW | 0 | 1: Enables MBOX_3_STAT to trigger interrupt if interrupt condition is met. 0: Disables MBOX_3_STAT interrupt. |
| #MBOX_2_STAT_EN | 2 | RW | 0 | Same as MBOX_3_STAT_EN |
| #MBOX_1_STAT_EN | 1 | RW | 0 | Same as MBOX_3_STAT_EN |
| #MBOX_0_STAT_EN | 0 | RW | 0 | Same as MBOX_3_STAT_EN |

## MBOX_STAT_INT_STAT_REG

Illustrates current interrupt status of mailboxes.

STAT_INT_STAT_REG is defined once in this document. All other STAT_INT_STAT_REG registers’ definition will be similar to this and their bits will follow their respective STAT_REG.

| #Register Name |  | MBOX_STAT_INT_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_INT_STAT | 3 | RO | 0 | 1: MBOX_3 INT is active and requesting service. 0: MBOX_3 INT is inactive. |
| #MBOX_2_INT_STAT | 2 | RO | 0 | Same as MBOX_3_INT_STAT |
| #MBOX_1_INT_STAT | 1 | RO | 0 | Same as MBOX_3_INT_STAT |
| #MBOX_0_INT_STAT | 0 | RO | 0 | Same as MBOX_3_INT_STAT |

## MBOX_STAT_INT_CLR_REG

Write one in this register to clear respective interrupt in MBOX_STAT_INT_STAT_REG. Applicable to Edge Interrupts.

STAT_INT_CLR_REG is defined once in this document. All other STAT_INT_CLR_REG registers’ definition will be similar to this and their bits will follow their respective STAT_REG.

| #Register Name |  | MBOX_STAT_INT_CLR_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #MBOX_3_INT_CLR | 3 | RW | 0 | One in this bit location clears MBOX_3_INT_STAT. This bit resets itself in next clock cycle after a one is written into it, so it will always show 0 if SW attempt to read it. |
| #MBOX_2_INT_CLR | 2 | RW | 0 | Same as MBOX_3_INT_CLR |
| #MBOX_1_INT_CLR | 1 | RW | 0 | Same as MBOX_3_INT_CLR |
| #MBOX_0_INT_CLR | 0 | RW | 0 | Same as MBOX_3_INT_CLR |

## SCRTCH_n_REG

| #Register Name |  | SCRTCH_n_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Scratch Pad |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #DATA | 31:0 | RW | 0 |  |

## SEM_REG

Here is how SEM_REG works:

- If the content of this register is zero, then it will allow SW to write any value on that.

- If the content of this register is nonzero, then SW is not allowed to write any value in this register but SW can clear this register by writing zero in it. In other words since this is 8-bit register, SW can clear this register by writing any value which has bits [7:0] = 8’h00. SW can always write zero in this register.

| #Register Name |  | SEM_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | All |  |  |
| Description |  | Semaphore |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #DATA | 7:0 | RW | 0 |  |

## SPI_CMD_STAT_REG

SW should set only single command bit at a time. If more than one command bits are set, only the command bit in lower bit position will be executed. For example of READ_DW and WRITE_DW commands are set, only READ_DW (which is in lower bit position) is executed.

| #Register Name |  | SPI_CMD_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Instruct FPGA to erase, read from, and write into SPI peripheral. |  |  |
| POR Value |  | 0x0000_0100 |  |  |
| Field | Bits | Type | Reset | Description |
| #INT_EN | 31 | RW | 0 | 1: Interrupt Enabled. |
| #RSVD | 30:12 | RO | 0 | Reserved |
| #INT | 11 | W1C | 0 | 1: SPI transaction is completed. |
| #ILLEGAL_ERASE | 10 | W1C | 0 | 1: ILLEGAL ERASE detected. Trying to erase a protected sector. In Virtual SPI Reg: Not Used. |
| #ILLEGAL_WRITE | 9 | W1C | 0 | 1: ILLEGAL WRITE detected. Trying to write in a protected sector. In Virtual SPI Reg: Not Used. |
| #READY | 8 | RO | 1 | 1: Flash memory controller is ready and can accept new command. 0: Flash memory controller is executing a command and cannot accept new command. Recommendation: SW should check this bit before initiating a new command. |
| #READ_ID | 7 | RW | 0 | 1: Read Device Identification from the memory device 0: No Action This bit is not used for internal flash access in PMC, ICB-FT and ICB-RDC FPGAs. In Virtual SPI Reg: Not Used. |
| #READ_SILICON_ID | 6 | RW | 0 | Not used in Ridley. Use bit 7 to read silicon ID. |
| #READ_STATUS | 5 | RW | 0 | This is to read Status Register-1 from flash. This bit is used differently in VSPI flash. In Virtual SPI Flash this bit is W1C. 1: Read Failed 0: Read Successful In Virtual SPI Reg this bit is not used. |
| #PROTECT_SECTOR | 4 | RW | 0 | Not used in Ridley. |
| #ERASE_SECTOR | 3 | RW | 0 | 1: Erase a sector specified by SPI_ADD_REG. 0: No Action In Virtual SPI Reg: Not Used. |
| #WRITE_W | 2 | RW | 0 | 1: Write one DW in Flash Memory address specified by SPI_ADD_REG. The data to be written should be stored in SPI_WR_DATA_REG. 0: No Action |
| #READ_W | 1 | RW | 0 | 1: Read one DW from address specified by SPI_ADD_REG. The read data will be stored in SPI_RD_DATA_REG. 0: No Action |
| #NEW_COMMAND | 0 | RW | 0 | 1: The command specified in Bit1 to 7 is a new command. 0: Command has been executed by FPGA and if there is any command bit set, FPGA will assume it is old command and will ignore it. To initiate a new command (specified by bit1 to bit7), SW should set NEW_COMMAND bit high. This bit is de-asserted by FPGA after command is executed. For Virtual SPI, if SW trigger a SPI command and the NEW_COMMAND bit never get cleared by FPGA (due to errors on the LSB bus) then SW can clear NEW_COMMAND bit and initiate another SPI command. In such a condition, |

## SPI_ADD_REG

In SPI controller and in VSPI-Flash this register is used as it is defined in the description field below, but in VSPI-WB the upper 4 bits are used differently. In VSPI_WB, this register is used to specify the address of the register in the remote FPGA. Bits [19:0] are used as register offset (relative address in the respective function). Bits [23:20] are used to specify the function.

Bits [23:20] = 0x1 and 0x0 specifies function 0.

Bits [23:20] = 0x2 specifies function 1.

Bits [23:20] = 0x4 specifies function 2.

Bits [23:20] = 0x8 specifies function 3.

Other values for Bits[23:20] are invalid.

| #Register Name |  | SPI_ADD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Specifies an address in SPI Peripheral |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:24 | RO | 0 | Reserved |
| #ADDR | 23:0 | RW | 0 | This field specifies the word address for READ_DW and WRITE_DW operation. A valid word address should have two LSB bits of zero. For sector erase command, the address placed in this field can be any valid address in the sector that you can erase. |

## SPI_WR_DATA_REG

| #Register Name |  | SPI_WR_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Data to be written in SPI Peripheral. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #WR_DATA | 31:0 | RW | 0 | 32-bit data to be written in Configuration Flash Memory. |

## SPI_RD_DATA_REG

| #Register Name |  | SPI_RD_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Data read from SPI Peripheral |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RD_DATA | 31:0 | RO | 0 | FPGA stores read data from configuration flash memory in this register. The register content is valid after Read command is executed and will remain valid before the next command starts, so SW is expected to read this register after read command executed before issuing another command. |

## SPI_RST_REG

| #Register Name |  | SPI_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 |  |
| #LCL_RST | 3:0 | RW | 0 | 0xD: Reset the local SPI controller or local VSPI module. With reset all registers are initialized by their POR value. All state machines are initialized to their initial state. After successful reset of SPI controller, this register will also be cleared to zero. Any other values other than 0xD will be ignored by FPGA. |

## I2C_PRSCL_LO_REG

Pre-scale value is calculated as: [(input clock HZ) / (5xdesired SCL HZ)] – 1.

Below values have been calculated based on 75MHZ input clock.

Pre-scale = 0x88 (136 decimal) for 100KHZ SCL

Pre-scale = 0x24 (36 decimal) for 400KHZ SCL

Change the value of the prescale register only when the I2C_MOD_EN bit is cleared.

| #Register Name |  | I2C_PRSCL_LO_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  | This register is used to pre-scale the SCL clock line |  |  |
| POR Value |  | 0x0000_00FF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #PRESCALE | 7:0 | RW | 0xFF | Clock Pre scale register lo-byte |

## I2C_PRSCL_HI_REG

| #Register Name |  | I2C_PRSCL_HI_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  | This register is used to pre-scale the SCL clock line |  |  |
| POR Value |  | 0x0000_00FF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #PRESCALE | 7:0 | RW | 0xFF | Clock Pre scale register hi-byte |

## I2C_CTRL_REG

| #Register Name |  | I2C_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #I2C_MOD_EN | 7 | RW | 0 | “1” means core is enabled. “0” means core is disabled. |
| #I2C_MOD_INT_EN | 6 | RW | 0 | “1” means interrupt is enabled. “0” means interrupt is disabled. |
| #RSVD | 5:0 | RO | 0 | Reserved |

## I2C_TX_REG

This is Transmit Register during register write operation but it will be Receive Register during register read operation.

| #Register Name |  | I2C_TX_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  | Transmit Data Register |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #DATA_BYTE_TX | 7:0 | WO | 0x0 | Next byte to be transmitted by I2C. In case of data transfer, bit [0] represents the data’s LSB. In case of a slave address transfer bit [0] represents the RW bit. ‘1’ means reading from slave and ‘0’ means writing to slave. |

## I2C_RX_REG

| #Register Name |  | I2C_RX_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  | Receive Data Register |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #DATA_BYTE_RX | 7:0 | RO | 0x00 | Last byte received via I2C |

## I2C_CMD_REG

This is command register during register write operation. During register read it will be status register.

Command bits [0] and [4:7] are cleared when I2C byte transaction is done or when bus arbitration is lost.

Prior to setting a command SW has to make sure right values have already been written in pre‐scale, Ctrl, Mux, and TX registers

| #Register Name |  | I2C_CMD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #STA | 7 | WO | 0 | generate (repeated) start condition |
| #STO | 6 | WO | 0 | generate stop condition |
| #RD | 5 | WO | 0 | read from slave |
| #WR | 4 | WO | 0 | write to slave |
| #ACK | 3 | WO | 0 | when a receiver, sent ACK (ACK = ‘0’) or NACK (ACK = ‘1’) |
| #RSVD | 2:1 | RO | 0 | Reserved |
| #INT_ACK | 0 | WO | 0 | Interrupt acknowledge. When set, clears a pending interrupt. |

## I2C_STAT_REG

| #Register Name |  | I2C_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #RX_ACK | 7 | RO | 0 | Received acknowledge from slave. This flag represents acknowledge from the addressed slave. ‘1’ = No acknowledge received ‘0’ = Acknowledge received |
| #BUSY | 6 | RO | 0 | I2C bus busy ‘1’ after START signal detected ‘0’ after STOP signal detected |
| #ARB_LOST | 5 | RO | 0 | Arbitration lost. This bit is set when the core lost arbitration. Arbitration is lost when: • a STOP signal is detected, but not requested • The master drives SDA high, but SDA is low. |
| #RSVD | 4:2 | RO | 0 | write to slave |
| #TIP | 1 | RO | 0 | Transfer in progress ‘1’ when transferring data ‘0’ when transfer complete |
| #INT_FLAG | 0 | RO | 0 | This bit is set when an interrupt is pending, which will cause a processor interrupt request if the IEN (INT_EN) bit is set. The Interrupt Flag is set when: • one byte transfer has been completed • arbitration is lost |

## I2C_MUX_SEL_REG

If a single I2C port is used to communicate with multiple I2C devices, then SW needs to set the device number in this register to specify which device it wants to talk to. FPGA will use this register to set the right values for the Enable and Select pins of the external I2C multiplexers.

If there is no external I2C multiplexer connected to I2C controller, value of this register is simply ignored and has no effect in I2C controller functions.

| #Register Name |  | I2C_MUX_SEL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 |  |
| #DEV_SEL | 7:0 | RW | 0 | Specifies the I2C device that needs to be connected to I2C controller. FPGA use this value to set the enable and select pins for the external I2C multiplexer. |

## I2C_RST_REG

| #Register Name |  | I2C_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 3, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 |  |
| #LCL_RST | 3:0 | RW | 0 | 0xD: Reset the local I2C controller or local VI2C module. With reset all registers are initialized by their POR value. All state machines are initialized to their initial state. After successful reset of I2C controller, this register will also cleared to zero. Any other values other than 0xD will be ignored by FPGA. |

## LED_SYNC_REG

| #Register Name |  | LED_SYNC_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 0, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 |  |
| #LED_SYNC | 0 | RW | 0 | This is used to synchronize the LED blinks on all the boards in chassis. SW write 1 in this bit to synchronize the LEDs. 1: Resets blinking status of all the Leds 0: no Action This bit is automatically cleared by FPGA. |

## LED_CONFIG_REG

| #Register Name |  | LED_CONFIG_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | ALL |  |  |
| FPGA Type |  | ALL |  |  |
| Function |  | 0, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0003_0705 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:28 | RO | 0 | Reserved |
| #LED_TMR_CTRL | 27:24 | RW | 0 | 0x09: Turns of the LED timer Other values: no effect. This field is only used at 300 sec after power up or after FPGA reconfiguration. SW has to write 0x9 in this field within 300 sec after power up (or after FPGA reconfiguration) to illustrate that it is in control otherwise FPGA will change the status of HEALTH LED to Orange Slow blink. After FPGA changed the status of HEALTH LED, SW still can change the status of the LED by writing in respective LED control bits in LED control register. |
| #RSVD | 23:21 | RO | 0 | Reserved |
| #FST_BLNK_SPD | 20:16 | RW | 0x3 | The value of this field is used only if LED is in fast blink mode. 0: 100ms 1: 200ms 3: 400ms … 31: 3200ms (3.2 sec) |
| #RSVD | 15:13 | RO | 0 |  |
| #SLW_BLNK_SPD | 12:8 | RW | 0x7 | The value of this field is used only if LED is in slow blink mode. 0: 100ms 1: 200ms 3: 400ms … 31: 3200ms (3.2 sec) |
| #RSVD | 7:4 | RO | 0 |  |
| #BRGHT_LVL | 3:0 | RW | 0x5 | The value of this field is used only when we put LED in half-bright mode in LED_CTRL_REG. 0: off 1: 10% of full brightness 2: 20% of full brightness …. 9: 90% of full brightness 10 to 15: Full bright |

## FPGA_RECONFIG_CTRL_REG

| #Register Name |  | FPGA_RECONFIG_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0, none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:9 | RO | All 0s | Reserved |
| #SELECT_IMAGE | 8 | RW | 0 | 0: Boot from App1 1: Boot from App2 |
| #UPDATE_FIRST_BOOT | 7:4 | RW | 0 | 0xD: Update ‘First Boot’ value in the Flash to the value shown by #SELECT_IMAGE Any other value: Does nothing. This field will get cleared by FPGA after command is executed successfully. |
| #START_RECONFIG | 3:0 | RW | 0 | 0xD: Start FPGA reconfiguration Any other value: Does nothing. |

## FPGA_RECONFIG_STAT_REG

| #Register Name |  | FPGA_RECONFIG_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | All |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0002 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:5 | RO | All 0s | Reserved |
| #ERROR_FIRST_BOOT | 4 | RO | 0 | 1: Error in UPDATE_FIRST_BOOT command |
| #BUSY | 3 | RO | 0 | FPGA reconfiguration is busy and executing a command. |
| #ERROR | 2 | RO | 0 | 0: No Error in FPGA Configuration 1: Error observed in FPGA reconfiguration |
| #CURRENT_IMAGE | 1:0 | RO | 10 | 10: App1 11: App2 |

# CB Registers

All registers are not protected by default unless specified.

## Function 0

## FPGA_REV_ID_REG

## FUNC_CAP_0_REG

## FPGA_RECONFIG_CTRL_REG

## FPGA_RECONFIG_STAT_REG

## SCRTCH_n_REG

## DEV_MSI_0_PEND_REG (TBD)

| #Register Name |  | DEV_MSI_0_PEND_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #MBOX_INT | 0 | RO | 0 | 1: MBOX requires service. 0: no service is requested. |

## DEV_MSI_0_INT_EN_REG (TBD)

| #Register Name |  | DEV_MSI_0_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 | Reserved |
| #MBOX_INT_EN | 0 | RW | 0 | 1: MBOX interrupt is enabled. 0: MBOX interrupt is disabled. |

## PWR_CTRL_REG

| #Register Name |  | PWR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0009 |  |  |
| Field | Bits | Type | Reset | Description |
| #PWR_SUICIDE | 31:16 | RW | All 0s | Value of 0xA73D in this register will disable 1Khz signal sent out to the hardware external watchdog and toggle the reconfiguration pin of CB. This field is used for CB post-ISP update. |
| #THERMAL_SUICIDE_SWB_1 | 15:12 | RW | All 0s | 0x6: Force SWB 1 PSSM to enter Thermal Fault state, CB will shutdown all power rails and recovers after 5min. SW writes to this register when it detects over-temperature event. Any other value: no effect, FPGA will not take any action. This bit is automatically cleared to 0 after shutdown. |
| #THERMAL_SUICIDE_SWB_0 | 11:8 | RW | All 0s | 0x6: Force SWB 0 PSSM to enter Thermal Fault state, CB will shutdown all power rails and recovers after 5min. SW writes to this register when it detects over-temperature event. Any other value: no effect, FPGA will not take any action. This bit is automatically cleared to 0 after shutdown. |
| #THERMAL_SUICIDE_MAIN | 7:4 | RW | 0x0 | 0x6: Force Main PSSM to enter Thermal Fault state, CB will shutdown all power rails and recovers after 5min. SW writes to this register when it detects over-temperature event. Any other value: no effect, FPGA will not take any action. This bit is automatically cleared to 0 after shutdown. |
| #EN_ISP_MODE | 3:0 | RW | 0x9 | 0x6: Enter ISP mode. 0x9: Disable ISP mode. Although any value can be written in this field by SW, FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. When enabled, this disable the power sequence logic of CB from monitoring the power good of PMC, SWB and ICB that is invalid when the PMC, SWB or ICB are performing self-reconfiguration. |

## PWR_STAT_REG

| #Register Name |  | PWR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:11 | RO | All 0s | Reserved |
| #SWB_1_ALL_PG | 10 | RO | 1 | 1: Pgood is okay |
| #SWB_0_ALL_PG | 9 | RO | 1 | 1: Pgood is okay |
| #PDB_ALL_PG | 8 | RO | 1 | 1: Pgood is okay |
| #RSVD | 7 | RO | 0 | Reserved |
| #SWB1_PWROK | 6 | RO | 0 | Indicates the power sequencing of SWB 1 is completed. 1: Asserted 0: De-asserted |
| #SWB0_PWROK | 5 | RO | 0 | Indicates the power sequencing of SWB 0 is completed. 1: Asserted 0: De-asserted |
| #CB_PWROK | 4 | RO | 0 | Indicates the power sequencing of Control Board is completed. 1: Asserted 0: De-asserted |
| #RSVD | 3:1 | RO | 0 | Reserved |
| #PMC_PWR_INT | 0 | RO | 0 | PMC Power interrupt 1: Asserted 0: De-asserted (signal is buffered from PMC) |

## RS232_SPD_SNS_REG

This register allows the sensing of the console baud rate. A write to this register will reset the counter and arm the speed sense circuit. When a character is sensed, the counter counts to the end of the start bit. At this point an interrupt is set and the count can be read to allow the baud rate to be determined. Upon writing this register back to zero, you clear the interrupt and send the state machine back to the wait for character state.

| #Register Name |  | RS232_SPD_SNS_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  | This register allows the sensing of the console baud rate |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:18 | RO | All 0s | Reserved |
| #SPEED_SENSE | 17:0 | RW | 0x00000 | Number of clocks the start bit is low. A character with odd ASCII code (e.g. Carriage Return) should be selected for speed sense. |

## RS232_CLR_SPD_SNS_REG

Speed Sense Interrupt will be reset by writing to this register.

| #Register Name |  | RS232_CLR_SPD_SNS_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  | Clear Speed Sense Interrupt |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:0 | RO |  | Write any value to clear the Speed Sense Interrupt |

## TPM_STAT_REG

| #Register Name |  | TPM_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #TPM_INT | 0 | RO | 0 | 1: Request for Service (signal is buffered from PMC) |

## PMC_CTRL_REG

| #Register Name |  | PMC_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:12 | RO | All 0s | Reserved |
| #PMC_LSB_RST | 11:8 | RW | 0 | Reset control of PMC LSB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will reset the PMC LSB port and all related processors. This Field is automatically cleared by FPGA after reset is issued. |
| #RSVD | 7:0 | RO | 0 | Reserved |

## PMC_STAT_REG

| #Register Name |  | PMC_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:28 | RO | All 0s | Reserved |
| #LSB_PROC_TYP_ERR | 27 | RO | 0 | One of the processors in LSB port received a packet with an invalid value in type field. |
| #LSB_PROC_FATAL_ERR | 26 | RO | 0 | One of the state machines in one of the processors in LSB port has gone momentarily to unknown state. |
| #LSB_PROC_EOP_ERR | 25 | RO | 0 | One of the processors in LSB port received a packet without EOP. |
| #LSB_PROC_SOP_ERR | 24 | RO | 0 | One of the processors in LSB port received a packet without SOP. |
| #RSVD | 23:19 | RO | 0 | Reserved |
| #LSB_TX_FATAL_ERR | 18 | RO | 0 | One of the state machines in LSB TX PHY has gone momentarily to unknown state. |
| #LSB_TX_EOP_ERR | 17 | RO | 0 | LSB TX PHY received a packet without EOP from one of the processors. |
| #LSB_TX_SOP_ERR | 16 | RO | 0 | LSB TX PHY received a packet without SOP from one of the processors. |
| #RSVD | 15 | RO | 0 | Reserved |
| #LSB_RX_REV_NO_ERR | 14 | RO | 0 | REV of the received packet does not match with the REV_NO of the LSB RX PHY. |
| #LSB_RX_UNEXP_ERR | 13 | RO | 0 | Unexpected condition in LSB RX PHY. 1: means RX_BUF is almost full or LSB_RX_PHY_CTRL has received a packet without SOP from DES (deserializer) |
| #LSB_RX_SEQNO_ERR | 12 | RO | 0 | LSB RX PHY received a packet from remote party with invalid SEQ no. |
| #LSB_RX_DEV_ID_ERR | 11 | RO | 0 | LSB RX PHY received a packet from remote party with invalid DEV-ID. |
| #LSB_RX_FATAL_ERR | 10 | RO | 0 | One of the state machines in LSB RX PHY has gone momentarily to unknown state. |
| #LSB_RX_LNGTH_ERR | 9 | RO | 0 | LSB RX PHY received a packet from remote party with invalid Length. |
| #LSB_RX_CHKSM_ERR | 8 | RO | 0 | LSB RX PHY received a packet from remote party with checksum error. |
| #RSVD | 7:0 | RO | 0 | Reserved |

## ICB_CTRL_REG

| #Register Name |  | ICB_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:24 | RO | All 0s | Reserved |
| #HSC_ENC_LATCH_OFF | 23:21 | RW | 0x0 | Send special Encoded Reset to ICB to hotswap controllers LATCH_OFF signal 0xD: Asserted Any other value: de-asserted. When asserted by SW, will power OFF system until switch tray removal. This Field is automatically cleared by FPGA after reset is issued. |
| #HSC_ENC_RST | 19:16 | RW | 0x0 | Send special Encoded Reset to ICB to toggle hotswap controllers 0xD: Asserted Any other value: de-asserted. When asserted by SW, will cycle the system power. This Field is automatically cleared by FPGA after reset is issued. |
| #RSVD | 15:12 | RO | All 0s | Reserved |
| #ICB_LSB_RST | 11:8 | RW | 0x0 | Reset control of ICB LSB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will reset the ICB LSB port and all related processors. This Field is automatically cleared by FPGA after reset is issued. |
| #RSVD | 7:4 | RO | 0 | Reserved |
| #ICB_ERST | 3:0 | RW | 0 | Reset control of ICB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will send encoded reset to the SWB on the respective switch board. The reset will be sent once, when this field is set to 0xD. This Field is automatically cleared by FPGA after encoded reset is issued. |

## ICB_STAT_REG

| #Register Name |  | ICB_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0002 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:28 | RO | All 0s | Reserved |
| #LSB_PROC_TYP_ERR | 27 | RO | 0 | One of the processors in LSB port received a packet with an invalid value in type field. |
| #LSB_PROC_FATAL_ERR | 26 | RO | 0 | One of the state machines in one of the processors in LSB port has gone momentarily to unknown state. |
| #LSB_PROC_EOP_ERR | 25 | RO | 0 | One of the processors in LSB port received a packet without EOP. |
| #LSB_PROC_SOP_ERR | 24 | RO | 0 | One of the processors in LSB port received a packet without SOP. |
| #RSVD | 23:19 | RO | 0 | Reserved |
| #LSB_TX_FATAL_ERR | 18 | RO | 0 | One of the state machines in LSB TX PHY has gone momentarily to unknown state. |
| #LSB_TX_EOP_ERR | 17 | RO | 0 | LSB TX PHY received a packet without EOP from one of the processors. |
| #LSB_TX_SOP_ERR | 16 | RO | 0 | LSB TX PHY received a packet without SOP from one of the processors. |
| #RSVD | 15 | RO | 0 | Reserved |
| #LSB_RX_REV_NO_ERR | 14 | RO | 0 | REV of the received packet does not match with the REV_NO of the LSB RX PHY. |
| #LSB_RX_UNEXP_ERR | 13 | RO | 0 | Unexpected condition in LSB RX PHY. 1: means RX_BUF is almost full or LSB_RX_PHY_CTRL has received a packet without SOP from DES (deserializer) |
| #LSB_RX_SEQNO_ERR | 12 | RO | 0 | LSB RX PHY received a packet from remote party with invalid SEQ no. |
| #LSB_RX_DEV_ID_ERR | 11 | RO | 0 | LSB RX PHY received a packet from remote party with invalid DEV-ID. |
| #LSB_RX_FATAL_ERR | 10 | RO | 0 | One of the state machines in LSB RX PHY has gone momentarily to unknown state. |
| #LSB_RX_LNGTH_ERR | 9 | RO | 0 | LSB RX PHY received a packet from remote party with invalid Length. |
| #LSB_RX_CHKSM_ERR | 8 | RO | 0 | LSB RX PHY received a packet from remote party with checksum error. |
| #RSVD | 7:3 | RO | 0 | Reserved |
| #ICB_INT | 2 | RO | 0 | INT request of PDB board 1: ICB requests interrupt 0: No interrupt requested This bit stores the filtered status of the pin |
| #ICB_RDY | 1 | RO | 1 | Ready status of ICB FPGA 1: ICB is Ready 0: ICB is Not Ready This bit stores the filtered status of the pin. |
| #RSVD | 1:0 | RO | 0 | Reserved |

## SWB_CTRL_REG

| #Register Name |  | SWB_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:12 | RO | All 0s | Reserved |
| #SWB_LSB_RST | 11:8 | RW | 0 | Reset control of SWB_FPGA LSB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will reset the SWB_FPGA LSB port and all related processors. This Field is automatically cleared by FPGA after reset is issued. |
| #SWB_PERST | 7:4 | RW | All 0s | PCIE Reset control of SWB 0xD: Asserted Any other value: De-asserted This Field is NOT automatically cleared by FPGA after reset is issued. |
| #SWB_ERST | 3:0 | RW | 0 | Reset control of SWB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will send encoded reset to the SWB on the respective switch board. The reset will be sent once, when this field is set to 0xD. This Field is automatically cleared by FPGA after encoded reset is issued. |

## SWB_STAT_REG

| #Register Name |  | SWB_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0001 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:28 | RO | All 0s | Reserved |
| #LSB_PROC_TYP_ERR | 27 | RO | 0 | One of the processors in LSB port received a packet with an invalid value in type field. |
| #LSB_PROC_FATAL_ERR | 26 | RO | 0 | One of the state machines in one of the processors in LSB port has gone momentarily to unknown state. |
| #LSB_PROC_EOP_ERR | 25 | RO | 0 | One of the processors in LSB port received a packet without EOP. |
| #LSB_PROC_SOP_ERR | 24 | RO | 0 | One of the processors in LSB port received a packet without SOP. |
| #RSVD | 23:19 | RO | 0 | Reserved |
| #LSB_TX_FATAL_ERR | 18 | RO | 0 | One of the state machines in LSB TX PHY has gone momentarily to unknown state. |
| #LSB_TX_EOP_ERR | 17 | RO | 0 | LSB TX PHY received a packet without EOP from one of the processors. |
| #LSB_TX_SOP_ERR | 16 | RO | 0 | LSB TX PHY received a packet without SOP from one of the processors. |
| #RSVD | 15 | RO | 0 | Reserved |
| #LSB_RX_REV_NO_ERR | 14 | RO | 0 | REV of the received packet does not match with the REV_NO of the LSB RX PHY. |
| #LSB_RX_UNEXP_ERR | 13 | RO | 0 | Unexpected condition in LSB RX PHY. 1: means RX_BUF is almost full or LSB_RX_PHY_CTRL has received a packet without SOP from DES (deserializer) |
| #LSB_RX_SEQNO_ERR | 12 | RO | 0 | LSB RX PHY received a packet from remote party with invalid SEQ no. |
| #LSB_RX_DEV_ID_ERR | 11 | RO | 0 | LSB RX PHY received a packet from remote party with invalid DEV-ID. |
| #LSB_RX_FATAL_ERR | 10 | RO | 0 | One of the state machines in LSB RX PHY has gone momentarily to unknown state. |
| #LSB_RX_LNGTH_ERR | 9 | RO | 0 | LSB RX PHY received a packet from remote party with invalid Length. |
| #LSB_RX_CHKSM_ERR | 8 | RO | 0 | LSB RX PHY received a packet from remote party with checksum error. |
| #RSVD | 7:3 | RO | 0 | Reserved |
| #SWB_INT | 2 | RO | 0 | INT request of switch board 1: SWB requests interrupt 0: No interrupt requested This bit stores the filtered status of the pin |
| #RSVD | 1 | RO | 0 | Reserved |
| #SWB_PRSNT | 0 | RO | 1 | Present status of switch board 1: SWB is present 0: SWB is Not Present This bit stores the filtered status of the pin. |

## BMC_CTRL_REG

| #Register Name |  | BMC_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0160 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:9 | RO | All 0s | Reserved |
| #BMC_BOOT_SEL | 8 | RW | 1 | Boot selection for BMC 1: SPI flash 0: UART |
| #BMC_PWR_EN | 7:4 | RW | 0x6 | Power control of BMC 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #BMC_RST | 3:0 | RW | 0x0 | Reset control of BMC 0xD: Asserted Any other value: de-asserted. When asserted by SW, will reset the BMC module. This Field is automatically cleared by FPGA after reset is issued. |

## BMC_STAT_REG

| #Register Name |  | BMC_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:5 | RO | All 0s | Reserved |
| #BMC_WDT_RST | 4 | RO | 0 | BMC Watchdog reset 1: Asserted 0: De-asserted |
| #RSVD | 3:1 | RO | All 0s | Reserved |
| #BMC_PRSNT | 0 | RO | 0 | Present status of BMC 1: Present 0: Not present |

## THRM_STAT_REG

| #Register Name |  | THRM_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:7 | RO | All 0s | Reserved |
| #TEMP_SNSR_TMP432A_ALERT | 6 | RO | 0 | On-board Temp Sensor (TM432A) alert, monitor location A,B and E (see ES for details) 1: Asserted 0: De-asserted |
| #TEMP_SNSR_TMP75_D_ALERT | 5 | RO | 0 | On-board Temp Sensor (TMP75) alert, monitor location D (see ES for details) 1: Asserted 0: De-asserted |
| #TEMP_SNSR_TMP75_C_ALERT | 4 | RO | 0 | On-board Temp Sensor (TMP75) alert, monitor location C (see ES for details) 1: Asserted 0: De-asserted |
| #RSVD | 3:2 | RO | All 0s | Reserved |
| #CPU_EXT_TEMP_SNSR_ALERT | 1 | RO | 0 | CPU External Temp Sensor (TMP461) alert, monitor CPU local, PMC and SSD temperature 1: Asserted 0: De-asserted (signal is buffered from PMC) |
| #CPU_INT_THRM_TRIP | 0 | RO | 0 | CPU over temperature and unconditional shutdown 1: Asserted 0: De-asserted (signal is buffered from PMC) |

## USB_CTRL_REG

| #Register Name |  | USB_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0006 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #USB_5V_EN | 3:0 | RW | 0x6 | Enable for USB 5V current limit switch 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## USB_STAT_REG

| #Register Name |  | USB_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #USB_VBUS_DET | 1 | RO | 0 | 1: USB VBUS detected 0: USB VBUS not detected (signal is buffered from PMC) |
| #USB_5V_FAULT | 0 | RO | 0 | 1: USB 5V fault is asserted 0: USB 5V fault is not asserted (signal is buffered from PMC) |

## BTN_STAT_REG

| #Register Name |  | BTN_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #FP_BTN | 0 | RO | 0 | Front Panel button is pressed. 1: Asserted 0: De-asserted |

## LEAK_STAT_REG

| #Register Name |  | LEAK_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #LEAK_ALERT | 1 | RO | 0 | Leak ADC module Alert 1: Asserted 0: De-asserted |
| #LEAK_ADC_INT | 0 | RO | 0 | Leak ADC module Interrupt 1: Interrupt asserted 0: Interrupt de-asserted |

## NFC_CTRL_REG

| #Register Name |  | NFC_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #NFC_EN | 3:0 | RO | 0 | Enable (Wake) signal of NFC module 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## NFC_STAT_REG

| #Register Name |  | NFC_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0001 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:3 | RO | All 0s | Reserved |
| #NFC_INT | 2 | RO | 0 | INT request of NFC module 1: NFC requests interrupt 0: No interrupt requested This bit stores the filtered status of the pin |
| #RSVD | 1 | RO | 0 | Reserved |
| #NFC_PRSNT | 0 | RO | 1 | Present status of NFC module 1: NFC is present 0: NFC is Not Present This bit stores the filtered status of the pin. |

## INTRD_STAT_REG

| #Register Name |  | INTRD_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  | Intrusion detection status |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #HANDLE_INT | 1 | RO | 0 | Interrupt of tray service or removal intent 1: Interrupt is asserted 0: Interrupt is not asserted |
| #COVER_INT | 0 | RO | 0 | Interrupt of top cover 1: Interrupt is asserted 0: Interrupt is not asserted |

## BTRY_STAT_REG

| #Register Name |  | BTRY_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #BTRY_UDV_ALERT | 0 | RO | 0 | Battery Under-voltage alert 1: Asserted 0: De-asserted |

## EN_ACCSS_PRTCTD_REG

This register is to protect critical registers from accidental writes. SW should write 0x8F35_D72C in order to have write access to protected registers.

| #Register Name |  | EN_ACCSS_PRTCTD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | PMC |  |  |
| Function |  | None |  |  |
| Description |  | Enables access to protected registers. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #EN_ACCSS | 31:0 | RW | 0 | 0x8F35_D72C: Enables access to protected registers. Other Values: Disables access to protected registers. |

## UART_MUX_REG

| #Register Name |  | UART_MUX_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:26 | RO | All 0s | Reserved |
| #SWB1_MUX_SEL | 25:24 | RW | 0x0 | SWB1 MUX select 3: UART Channel 4 2: UART Channel 3 1: UART Channel 2 0: UART Channel 1 |
| #RSVD | 23:22 | RO | All 0s | Reserved |
| #SWB0_MUX_SEL | 21:20 | RW | 0x0 | SWB0 MUX select 3: UART Channel 4 2: UART Channel 3 1: UART Channel 2 0: UART Channel 1 |
| #RSVD | 19:18 | RO | All 0s | Reserved |
| #BMC_MUX_SEL | 17:16 | RW | 0x0 | BMC MUX select 2: SWB1 1: SWB0 0: USB Console |
| #RSVD | 15:14 | RO | All 0s | Reserved |
| #CPU_DBG_MUX_SEL | 13:12 | RW | 0x0 | CPU Debug MUX select 2: SWB1 1: SWB0 0: USB Console |
| #RSVD | 11:10 | RO | All 0s | Reserved |
| #CPU_CNSL_MUX_SEL | 9:8 | RW | 0x0 | CPU Console MUX select 2: SWB1 1: SWB0 0: USB Console |
| #RSVD | 7:5 | RO | All 0s | Reserved |
| #USB_CNSL_MUX_SEL | 4:0 | RW | 0x0 | USB Console MUX select 4: SWB1 3: SWB0 2: BMC 1: CPU Debug 0: CPU Console |

## I2C_MUX_RST_REG

| #Register Name |  | I2C_MUX_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_00DD |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #I2C_CH14_MUX_RST | 7:4 | RW | 0xD | Reset control of I2C CH14 MUX 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |
| #I2C_CH10_MUX_RST | 3:0 | RW | 0xD | Reset control of I2C CH10 MUX 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |

## CPU_INTR_CTRL_REG

| #Register Name |  | CPU_INTR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #CPU_INTR_2 | 1 | RW | 0 | Control interrupt 2 to CPU 1: Assert interrupt 0: De-assert interrupt |
| #CPU_INTR_1 | 0 | RW | 0 | Control interrupt 1 to CPU 1: Assert interrupt 0: De-assert interrupt |

## BMC_JTAG_CTRL_REG

| #Register Name |  | BMC_JTAG_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  | Controls the source to/fom JTAG pins of BMC |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #TDO | 7 | RO | 0 | Input, Connected to BMC TDO pin |
| #TDI | 6 | RW | 0 | Output Connected to BMC TDI pin |
| #TMS | 5 | RW | 0 | Output Connected to BMC TMS pin |
| #TCK | 4 | RW | 0 | Output Connected to BMC TCK pin |
| #RSVD | 3:1 | RO | All 0s | Reserved |
| #JTAG_SEL | 0 | RW | 0 | Select the source to BMC JTAG 0: On-board JTAG header 1: CB control with bit [6:4] |

## SWB_JTAG_CTRL_REG

| #Register Name |  | SWB_JTAG_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  | Controls the source to/fom JTAG pins of SWB |  |  |
| POR Value |  | 0x0000_0100 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:9 | RO | All 0s | Reserved |
| #TRST | 8 | RW | 1 | Output Connected to SWB TRST pin |
| #TDO | 7 | RO | 0 | Input, Connected to SWB TDO pin |
| #TDI | 6 | RW | 0 | Output Connected to SWB TDI pin |
| #TMS | 5 | RW | 0 | Output Connected to SWB TMS pin |
| #TCK | 4 | RW | 0 | Output Connected to SWB TCK pin |
| #RSVD | 3:0 | RO | All 0s | Reserved |

## FLSH_CTRL_REG

This register is clear under two conditions only.

- Upon assertion of global reset.

- Power cycle

| #Register Name |  | FLSH_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:12 | RO | All 0s | Reserved |
| #PHY2_RETIMER_WP | 11 | RW | 0 | PHY Retimer Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |
| #PHY2_I210_CTRLR_SPI_WP | 10 | RW | 0 | PHY I210 Ethernet Controller SPI Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |
| #PHY1_84991_WP | 9 | RW | 0 | PHY 84991 Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |
| #BMC_EEPRM_WP | 8 | RW | 0 | BMC EEPROM Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |
| #RSVD | 7:3 | RO | All 0s | Reserved |
| #ZEROIZATION_WP | 2 | RW | 0 | 1: Protected, Once asserted, cannot be de-asserted by SW. SW writes into this register bit during boot and use it as a gate for zeroization. This would block a user from running zeroization (or parts of it) in Service-OS and Product-OS. It also blocks them clearing a sticky bit. |
| #FPGA_FLSH_WP | 1 | RW | 0 | FPGA (CB) Flash Write Protect 1: Protected, Once asserted, FPGA will not issue any write/erase command to the flash. Once asserted, cannot be de-asserted by SW. |
| #CHSS_EEPRM_WP | 0 | RW | 0 | System Key EEPROM Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |

## LED_SYNC_REG

## LED_CONFIG_REG

## LED_CTRL_0_REG

For bit definitions please refer to the section “LED Control bits”.

| #Register Name |  | LED_CTRL_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:24 | RO | All 0s | Reserved |
| #UID | 23:16 | RW | 0x0 | Locator LED |
| #ALM_STAT | 15:8 | RW | 0x0 | Alarm status LED |
| #SYS_STAT | 7:0 | RW | 0x0 | System status LED |

## LED_CTRL_1_REG

| #Register Name |  | LED_CTRL_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #PSU_STAT | 7:0 | RW | 0x0 | Fan status LED |

## LED_CTRL_2_REG

| #Register Name |  | LED_CTRL_2_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #FAN_STAT | 7:0 | RW | 0x0 | Fan status LED |

## LED_CTRL_3_REG

| #Register Name |  | LED_CTRL_3_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #LEAK_STAT | 7:0 | RW | 0x0 | LEAK status LED |

## LED_CTRL_4_REG

| #Register Name |  | LED_CTRL_4_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:16 | RO | All 0s | Reserved |
| #PHY_84991_ACT_STAT | 15:8 | RW | 0x0 | PHY 84991 activity status bit [7:6] option available (see Chapter 5): 00 = SW control 01 = PHY 84991 LED 0 driver 10 = PHY 84991 LED 1 driver 11 = PHY 84991 LED 2 driver |
| #PHY_84991_LINK_STAT | 7:0 | RW | 0x0 | PHY 84991 link status bit [7:6] option available (see Chapter 5): 00 = SW control 01 = PHY 84991 LED 0 driver 10 = PHY 84991 LED 1 driver 11 = PHY 84991 LED 2 driver |

## LED_CTRL_5_REG

| #Register Name |  | LED_CTRL_5_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:16 | RO | All 0s | Reserved |
| #PHY_I210_ACT_STAT | 15:8 | RW | 0x0 | PHY I210 activity status bit [7:6] option available (see Chapter 5): 00 = SW control 01 = PHY I210 LED 0 driver 10 = PHY I210 LED 1 driver 11 = PHY I210 LED 2 driver |
| #PHY_I210_LINK_STAT | 7:0 | RW | 0x0 | PHY I210 link status bit [7:6] option available (see Chapter 5): 00 = SW control 01 = PHY I210 LED 0 driver 10 = PHY I210 LED 1 driver 11 = PHY I210 LED 2 driver |

## WDG_CTRL_REG

This register will be reset upon CPU POR reset.

| #Register Name |  | WDG_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | none |  |  |
| Description |  | Watchdog control register |  |  |
| POR Value |  | 0x3000_0FFF |  |  |
| Field | Bits | Type | Reset | Description |
| #WDG_INT_EN | 31 | RW | 0 | 1: Enables wdog interrupt. 0: Disables wdog interrupt. |
| #WDG_EN | 30:27 | RW | 0x6 | 0x6: Enables wdog counter. 0x9: Disables wdog counter. Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous on/off status. After CPU POR reset is asserted, the wdog counter enable status follows the behavior below: - POR value when the register bit WDG_EN_POR_MSK of TND_CTRL_REG is de-asserted. - Register bit WDG_EN when the WDG_EN_POR_MSK of TND_CTRL_REG is asserted. CPU POR reset will be asserted if WDG_EN is asserted and WDG_CNTR_VALUE = WDG_LNGTH |
| #RSVD | 26:12 | RO | 0 | Reserved |
| #WDG_LNGTH | 11:0 | RW | 0xFFF | Number of 100ms ticks before the watchdog triggers. FPGA will never allow 0x000 to be written in this field. If SW tries write 0x000 in this field, FPGA will write POR value in this register. Count Time (ms) 0x0 illegal value 0x1 100.0 0x2 200.0 ... ... |

## WDG_STAT_REG

| #Register Name |  | WDG_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | none |  |  |
| Description |  | Watchdog status register |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #WDG_INT | 31 | RO | 0 | 1: Asserted, wdog interrupt occurred. 0: De-asserted, no interrupt yet. WDG_INT will be asserted if WDG_INT_EN is asserted and WDG_CNTR_VALUE = WDG_LNGTH |
| #RSVD | 30:12 | RO | 0 | Reserved |
| #WDG_CNTR_VLUE | 11:0 | RO | 0 | Illustrates the current value of wdog counter. Watchdog counter value will keep incrementing if WDG_EN is asserted. Watchdog counter will stop incrementing if counter value gets equal to WDG_LNGTH. Watchdog counter will reset to zero if WDG_EN bit is de-asserted or CPU POR reset is asserted. |

## WDG_RST_REG

Any write to this register will reset the watchdog counter and de-assert the interrupt.

| #Register Name |  | WDG_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | none |  |  |
| Description |  | Watchdog reset register |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:0 | RO | 0 | Reserved |

## TND_CTRL_REG

## TND_STAT_REG

## TND_PWR_CTRL_REG

| #Register Name |  | TND_PWR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_01FF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:21 | RO | All 0s | Reserved |
| #PSSM_DBG_PIN_OVR | 20 | RW | 0 | Enable override of PSSM for all bits [14:2] below 1: Enable 0: Disable |
| #RSVD | 19:17 | RO | 0 | Reserved |
| #PSSM_PWR_EN_OVR | 16 | RW | 0 | Enable override of PSSM for all bits [8:0] below 1: Enable 0: Disable |
| #RSVD | 15 | RO | 0 | Reserved |
| #SWB1_BYPASSBOOT | 14 | RW | 0 | SWB1 Bypass Boot value |
| #SWB0_BYPASSBOOT | 13 | RW | 0 | SWB0 Bypass Boot value |
| #DBGBOOT | 12 | RW | 0 | Debugboot value |
| #RSVD | 11:9 | RO | All 0s | Reserved |
| #SWB1_PWREN | 8 | RW | 1 |  |
| #SWB1_STBYEN | 7 | RW | 1 |  |
| #SWB0_PWREN | 6 | RW | 1 |  |
| #SWB0_STBYEN | 5 | RW | 1 |  |
| #PHY2_1V8_PWREN | 4 | RW | 1 |  |
| #PHY2_0V8_AVDD_PWREN | 3 | RW | 1 |  |
| #PHY2_0V8_DVDD_PWREN | 2 | RW | 1 |  |
| #PHY_V3P3_PWREN | 1 | RW | 1 |  |
| #CPU_MDL_PWREN | 0 | RW | 1 | Power enable overrides 1: Enable 0: Disable |

## TND_PWR_STAT_REG

| #Register Name |  | TND_PWR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_01FF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:19 | RO | All 0s | Reserved |
| #DIP_SWB1_BYPASSBOOT | 18 | RO | 0 | SWB1 Bypass Boot DIP switch position |
| #DIP_SWB0_BYPASSBOOT | 17 | RO | 0 | SWB0 Bypass Boot DIP switch position |
| #DIP_DBGBOOT | 16 | RO | 0 | Debugboot DIP switch position |
| #RSVD | 15:9 | RO | All 0s | Reserved |
| #SWB1_ALL_PG | 8 | RO | 1 |  |
| #SWB1_STBY_PG | 7 | RO | 1 |  |
| #SWB0_ALL_PG | 6 | RO | 1 |  |
| #SWB0_STBY_PG | 5 | RO | 1 |  |
| #PHY2_1V8_PG | 4 | RO | 1 |  |
| #PHY2_0V8_AVDD_PG | 3 | RO | 1 |  |
| #PHY2_0V8_DVDD_PG | 2 | RO | 1 |  |
| #PHY_0V8_AVDD_PG | 1 | RO | 1 |  |
| #CPU_MDL_PG | 0 | RO | 1 | Pgood status |

## TND_CLK_CTRL_REG

| #Register Name |  | TND_CLK_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0006_6066 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:20 | RO | All 0s | Reserved |
| #BUF2_En1 | 19:16 | RW | 0x6 | Clock Buffer 2 Output Enable 1 |
| #BUF2_En0 | 15:12 | RW | 0x6 | Clock Buffer 2 Output Enable 0 |
| #RSVD | 11:8 | RO | All 0s | Reserved |
| #BUF1_En1 | 7:4 | RW | 0x6 | Clock Buffer 1 Output Enable 1 |
| #BUF1_En0 | 3:0 | RW | 0x6 | Clock Buffer 1 Output Enable 0 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## TND_PMC_GPIO_CTRL_REG

| #Register Name |  | TND_PMC_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:15 | RO | 0 | Reserved |
| #GPI_14 | 14 | RW | 0 |  |
| #GPI_13 | 13 | RW | 0 |  |
| #GPI_12 | 12 | RW | 0 |  |
| #GPI_11 | 11 | RW | 0 |  |
| #GPI_10 | 10 | RW | 0 |  |
| #GPI_9 | 9 | RW | 0 |  |
| #GPI_8 | 8 | RW | 0 |  |
| #GPI_7 | 7 | RW | 0 |  |
| #GPI_6 | 6 | RW | 0 |  |
| #GPI_5 | 5 | RW | 0 | 1: Assert GPIO 5 0: De-assert GPIO 5 |
| #RSVD | 4:0 | RO | 0 | Reserved |

## TND_PMC_GPIO_STAT_REG

| #Register Name |  | TND_PMC_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:5 | RO | All 0s | Reserved |
| #GPO_14 | 14 | RO | 0 |  |
| #GPO_13 | 13 | RO | 0 |  |
| #GPO_12 | 12 | RO | 0 |  |
| #GPO_11 | 11 | RO | 0 |  |
| #GPO_10 | 10 | RO | 0 |  |
| #GPO_9 | 9 | RO | 0 |  |
| #GPO_8 | 8 | RO | 0 |  |
| #GPO_7 | 7 | RO | 0 |  |
| #GPO_6 | 6 | RO | 0 |  |
| #GPO_5 | 5 | RO | 0 | 1: GPIO 5 is asserted 0: GPIO 5 is de-asserted |
| #RSVD | 4:0 | RO | All 0s | Reserved |

## TND_PMC_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_PMC_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:15 | RO | 0 | Reserved |
| #GPIO_DIR_14 | 14 | RW | 0 |  |
| #GPIO_DIR_13 | 13 | RW | 0 |  |
| #GPIO_DIR_12 | 12 | RW | 0 |  |
| #GPIO_DIR_11 | 11 | RW | 0 |  |
| #GPIO_DIR_10 | 10 | RW | 0 |  |
| #GPIO_DIR_9 | 9 | RW | 0 |  |
| #GPIO_DIR_8 | 8 | RW | 0 |  |
| #GPIO_DIR_7 | 7 | RW | 0 |  |
| #GPIO_DIR_6 | 6 | RW | 0 |  |
| #GPIO_DIR_5 | 5 | RW | 0 | 1: Indicate GPIO 5 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 5 is configured as input and register GPIO_STAT_REG store this input port value. |
| #RSVD | 4:0 | RO | 0 | Reserved |

## TND_BMC_GPIO_CTRL_REG

| #Register Name |  | TND_BMC_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:7 | RO | 0 | Reserved |
| #GPI_6 | 6 | RW | 0 |  |
| #GPI_5 | 5 | RW | 0 |  |
| #GPI_4 | 4 | RW | 0 |  |
| #GPI_3 | 3 | RW | 0 |  |
| #GPI_2 | 2 | RW | 0 |  |
| #RSVD | 1 | RO | 0 | Reserved |
| #GPI_0 | 0 | RW | 0 | 1: Assert GPIO 0 0: De-assert GPIO 0 |

## TND_BMC_GPIO_STAT_REG

| #Register Name |  | TND_BMC_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:7 | RO | All 0s | Reserved |
| #GPO_6 | 6 | RO | 0 |  |
| #GPO_5 | 5 | RO | 0 |  |
| #GPO_4 | 4 | RO | 0 |  |
| #GPO_3 | 3 | RO | 0 |  |
| #GPO_2 | 2 | RO | 0 |  |
| #RSVD | 1 | RO | 0 | Reserved |
| #GPO_0 | 0 | RO | 0 | 1: GPIO 0 is asserted 0: GPIO 0 is de-asserted |

## TND_BMC_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_BMC_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:7 | RO | 0 | Reserved |
| #GPIO_DIR_6 | 6 | RW | 0 |  |
| #GPIO_DIR_5 | 5 | RW | 0 |  |
| #GPIO_DIR_4 | 4 | RW | 0 |  |
| #GPIO_DIR_3 | 3 | RW | 0 |  |
| #GPIO_DIR_2 | 2 | RW | 0 |  |
| #RSVD | 1 | RO | 0 | Reserved |
| #GPIO_DIR_0 | 0 | RW | 0 | 1: Indicate GPIO 0 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 0 is configured as input and register GPIO_STAT_REG store this input port value. |

## TND_SWB_GPIO_CTRL_REG

| #Register Name |  | TND_SWB_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 | Reserved |
| #GPI_7 | 7 | RW | 0 |  |
| #GPI_6 | 6 | RW | 0 | 1: Assert GPIO 6 0: De-assert GPIO 6 |
| #RSVD | 5:0 | RO | 0 | Reserved |

## TND_SWB_GPIO_STAT_REG

| #Register Name |  | TND_SWB_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #GPO_7 | 7 | RO | 0 |  |
| #GPO_6 | 6 | RO | 0 | 1: GPIO 6 is asserted 0: GPIO 6 is de-asserted |
| #RSVD | 5:0 | RO | 0 | Reserved |

## TND_SWB_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_SWB_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 | Reserved |
| #GPIO_DIR_7 | 7 | RW | 0 |  |
| #GPIO_DIR_6 | 6 | RW | 0 | 1: Indicate GPIO 6 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 6 is configured as input and register GPIO_STAT_REG store this input port value. |
| #RSVD | 5:0 | RO | 0 | Reserved |

## TND_ICB_GPIO_CTRL_REG

| #Register Name |  | TND_ICB_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #GPI_3 | 3 | RW | 0 |  |
| #GPI_2 | 2 | RW | 0 |  |
| #GPI_1 | 1 | RW | 0 | 1: Assert GPIO 1 0: De-assert GPIO 1 |
| #RSVD | 0 | RO | 0 | Reserved |

## TND_ICB_GPIO_STAT_REG

| #Register Name |  | TND_ICB_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #GPO_3 | 3 | RO | 0 |  |
| #GPO_2 | 2 | RO | 0 |  |
| #GPO_1 | 1 | RO | 0 | 1: GPIO 1 is asserted 0: GPIO 1 is de-asserted |
| #RSVD | 0 | RO | 0 | Reserved |

## TND_ICB_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_ICB_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 0 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #GPIO_DIR_3 | 3 | RW | 0 |  |
| #GPIO_DIR_2 | 2 | RW | 0 |  |
| #GPIO_DIR_1 | 1 | RW | 0 | 1: Indicate GPIO 1 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 1 is configured as input and register GPIO_STAT_REG store this input port value. |
| #RSVD | 0 | RO | 0 | Reserved |

## RST_CAUSE_REG

This is a protected register. This is sticky and does not get reset under global reset condition.

| #Register Name |  | RST_CAUSE_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | None |  |  |
| Description |  | Monitor the main PSSM |  |  |
| POR Value |  | 0x0000_0001 |  |  |
| Field | Bits | Type | Reset | Description |
| #SW_RST_COOKIES | 31:16 | RW | All 0s | For SW to read and write. |
| #RSVD | 15:9 | RO | All 0s | Reserved. |
| #CLD_RST | 8 | W1C | 0 | This bit will be set if the System PGOOD is de-sserted from a good state. |
| #RSVD | 7:4 | RO | All 0s | Reserved. |
| #HW_THERM_RST | 3 | W1C | 0 | This bit will be set if the hardware thermal suicide is caused by CB. |
| #WDG_RST | 2 | W1C | 0 | This bit will be set if the CB internal watch dog timer is expired. |
| #SW_THERM_RST | 1 | W1C | 0 | This bit will be set when SW asserts thermal suicide by asserting #THERMAL_SUICIDE_MAIN field in PWR_CTRL_REG. |
| #PWR_ON_RST | 0 | W1C | 1 |  |

## RST_CAUSE_SWB_REG

This is a protected register. This is sticky and does not get reset under global reset condition.

| #Register Name |  | RST_CAUSE_SWB_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | None |  |  |
| Description |  | Monitor the SWB PSSM |  |  |
| POR Value |  | 0x0000_0001 |  |  |
| Field | Bits | Type | Reset | Description |
| #SW_RST_COOKIES | 31:16 | RW | All 0s | For SW to read and write. |
| #RSVD | 15:9 | RO | All 0s | Reserved. |
| #CLD_RST | 8 | W1C | 0 | This bit will be set if the SWB PGOOD is de-sserted from a good state. |
| #RSVD | 7:4 | RO | All 0s | Reserved. |
| #HW_THERM_RST | 3 | W1C | 0 | This bit will be set if the hardware thermal suicide is caused by SWB. |
| #RSVD | 2 | RO | 0 | Reserved. |
| #SW_THERM_RST | 1 | W1C | 0 | This bit will be set when SW asserts thermal suicide by asserting #THERMAL_SUICIDE_SWB_<SWB_NO> field in PWR_CTRL_REG. |
| #PWR_ON_RST | 0 | W1C | 1 |  |

## Function 1

## FPGA_REV_ID_REG

## FUNC_CAP_0_REG

## SCRTCH_n_REG

## DEV_MSI_0_PEND_REG (TBD)

| #Register Name |  | DEV_MSI_0_PEND_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #MBOX_INT | 0 | RO | 0 | 1: MBOX requires service. 0: no service is requested. |

## DEV_MSI_0_INT_EN_REG (TBD)

| #Register Name |  | DEV_MSI_0_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 | Reserved |
| #MBOX_INT_EN | 0 | RW | 0 | 1: MBOX interrupt is enabled. 0: MBOX interrupt is disabled. |

## FAN_CTRL_REG

| #Register Name |  | FAN_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0666 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:17 | RO | All 0s | Reserved |
| #ALL_FAN_FULL_SPD | 16 | RW | 0 | Control all Fans (1,2,3) to run at full speed 1: Full speed 0: Non full speed |
| #RSVD | 15:12 | RW | 0x0 | Reserved |
| #EFUSE_EN_FAN3 | 11:8 | RW | 0x6 | Enable of Fan3 EFUSE 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #EFUSE_EN_FAN2 | 7:4 | RW | 0x6 | Enable of Fan2 EFUSE 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #EFUSE_EN_FAN1 | 3:0 | RW | 0x6 | Enable of Fan1 EFUSE 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## FAN_STAT_REG

| #Register Name |  | FAN_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:13 | RO | All 0s | Reserved |
| #FAN_CTRLR_ALERT | 12 | RO | 0 | Fan controller alert 1: Asserted 0: De-asserted |
| #RSVD | 11 | RO | 0 | Reserved |
| #ALRT_FAN_3 | 10 | RO | 0 | EFUSE alert of Fan 3 1: Asserted 0: De-asserted |
| #ALRT_FAN_2 | 9 | RO | 0 | EFUSE alert of Fan 2 1: Asserted 0: De-asserted |
| #ALRT_FAN_1 | 8 | RO | 0 | EFUSE alert of Fan 1 1: Asserted 0: De-asserted |
| #RSVD | 7 | RO | 0 | Reserved |
| #PG_FAN_3 | 6 | RO | 0 | Power status of Fan 3 1: Ok 0: Not Ok |
| #PG_FAN_2 | 5 | RO | 0 | Power status of Fan 2 1: Ok 0: Not Ok |
| #PG_FAN_1 | 4 | RO | 0 | Power status of Fan 1 1: Ok 0: Not Ok |
| #RSVD | 3 | RO | 0 | Reserved |
| #PRSNT_FAN_3 | 2 | RO | 0 | Present status of Fan 3 1: Present 0: Not present This bit stores the filtered status of the pin. |
| #PRSNT_FAN_2 | 1 | RO | 0 | Present status of Fan 2 1: Present 0: Not present This bit stores the filtered status of the pin. |
| #PRSNT_FAN_1 | 0 | RO | 0 | Present status of Fan 1 1: Present 0: Not present This bit stores the filtered status of the pin. |

## RTM_CTRL_REG

| #Register Name |  | RTM_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0001_009D |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:17 | RO | All 0s | Reserved |
| #RTM_SERBOOT | 16 | RW | 1 | Enable of Serial Boot of Retimer 1: from SPI ROM 0: from MDIO |
| #RSVD | 15:8 | RO | All 0s | Reserved |
| #RTM_OSC_EN | 7:4 | RW | 0x9 | Enable of PHY 81381 retimer 156.25MHz clock 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #RTM_RST | 3:0 | RW | 0xD | Reset of PHY 81381 retimer 0xD: Asserted Any other value: de-asserted. This Field is automatically cleared by FPGA after reset is issued. |

## RTM_STAT_REG

| #Register Name |  | RTM_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #PHY_RTM_INT | 0 | RO | 0 | Interrupt of PHY 813881 retimer 1: Asserted 0: De-asserted |

## ETH_CTRLR_CTRL_REG

| #Register Name |  | ETH_CTRLR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:3 | RO | All 0s | Reserved |
| #SD_PIN2 | 2 | RW | 0x0 | SW defined output pin2 to I210 controller |
| #SD_PIN1 | 1 | RW | 0x0 | SW defined output pin1 to I210 controller |
| #SD_PIN0 | 0 | RW | 0x0 | SW defined output pin0 to I210 controller |

## ETH_CTRLR_STAT_REG

| #Register Name |  | ETH_CTRLR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #I210_WAKE_INT | 1 | RO | 0 | ‘Wake’ signal from I210 1: Asserted 0: De-asserted |
| #I210_ALRT_INT | 0 | RO | 0 | Alert from I210 1: Asserted 0: De-asserted |

## USRPRT_PWR_CTRL_REG

| #Register Name |  | USRPRT_PWR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0099 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #USRPRT_EFUSE_EN_P2 | 7:4 | RW | 0x9 | Enable EFUSE for UserPort 2 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #USRPRT_EFUSE_EN_P1 | 3:0 | RW | 0x9 | Enable EFUSE for UserPort 1 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## USRPRT_PWR_STAT_REG

| #Register Name |  | USRPRT_PWR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #EFUSE_PG_P2 | 1 | RO | 0 | EFUSE Pgood of UserPort 2 (QSFP) 1: Asserted 0: De-asserted |
| #EFUSE_PG_P1 | 0 | RO | 0 | EFUSE Pgood of UserPort 1 (QSFP) 1: Asserted 0: De-asserted |

## USRPRT_CTRL_0_REG

| #Register Name |  | USRPRT_CTRL_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #P2_RST | 1 | RW | 0 | QSFP UserPort 2 Reset |
| #P1_RST | 0 | RW | 0 | QSFP UserPort 1 Reset 1: Assert reset 0: De-assert reset |

## USRPRT_STAT_0_REG

| #Register Name |  | USRPRT_STAT_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #P2_INT | 1 | RO | 0 | QSFP UserPort 2 Interrupt 1: Asserted 0: De-asserted |
| #P1_INT | 0 | RO | 0 | QSFP UserPort 1 Interrupt 1: Asserted 0: De-asserted |

## USRPRT_CTRL_1_REG

| #Register Name |  | USRPRT_CTRL_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #P2_LP_MODE | 1 | RW | 0 | QSFP UserPort 2 LP Mode |
| #P1_LP_MODE | 0 | RW | 0 | QSFP UserPort 1 LP Mode 1: Assert 0: De-assert |

## USRPRT_STAT_1_REG

| #Register Name |  | USRPRT_STAT_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #P2_ PRSNT | 1 | RO | 0 | QSFP UserPort 2 Present 1: Asserted 0: De-asserted |
| #P1_PRSNT | 0 | RO | 0 | QSFP UserPort 1 Present 1: Asserted 0: De-asserted |

## PHY_CTRL_REG

| #Register Name |  | PHY_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 1 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #PHY_RST | 3:0 | RW | 0x0 | PHY 84991 Reset 0xD: Asserted Any other value: de-asserted. This Field is automatically cleared by FPGA after reset is issued. |

## Function 2

## FPGA_REV_ID_REG

## FUNC_CAP_0_REG

## SCRTCH_n_REG

## DEV_MSI_0_PEND_REG (TBD)

| #Register Name |  | DEV_MSI_0_PEND_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #MBOX_INT | 0 | RO | 0 | 1: MBOX requires service. 0: no service is requested. |

## DEV_MSI_0_INT_EN_REG (TBD)

| #Register Name |  | DEV_MSI_0_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 | Reserved |
| #MBOX_INT_EN | 0 | RW | 0 | 1: MBOX interrupt is enabled. 0: MBOX interrupt is disabled. |

## SPI_CMD_STAT_REG

## SPI_ADD_REG

## SPI_WR_DATA_REG

## SPI_RD_DATA_REG

## SPI_RST_REG

## GEN_SPI_RD_DATA_REG

| #Register Name |  | GEN_SPI_RD_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Data read from SPI Peripheral |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RD_DATA | 31:0 | RO | 0 | Data read back from SPI transaction. Number of bits based on #SPI_DATA_WIDTH The read back data is obtain each time write operation is carried out by writing into the GEN_SPI_WR_DATA_REG. |

## GEN_SPI_WR_DATA_REG

| #Register Name |  | GEN_SPI_WR_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Data to be written into SPI Peripheral. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #WR_DATA | 31:0 | RW | 0 | Data to be written. Number of bits written based on #SPI_DATA_WIDTH Writing into this register will initiate the Generic SPI Master to send out the SPI transaction to the SPI Slave. |

## GEN_SPI_STAT

| #Register Name |  | GEN_SPI_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Status register for Generic SPI Master for SPI interface |  |  |
| POR Value |  | 0x0000_0060 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 30:9 | RO | All 0s | Reserved |
| #ERR_BIT | 8 | RO | 0 | Indicate overrun error has occur. 1: Transmit overrun error or receive overrun error has occur. 0: No overrun error has occurred. Recommendation: SW should check this bit before initiating a new command. |
| #RRDY | 7 | RO | 0 | Indicate receive ready status. 1: SPI_RD_DATA_REG register contains valid data and is ready to be read. 0: SPI_RD_DATA_REG register does not contain valid data. Reading the SPI_RD_DATA_REG clears this register to 0. |
| #TRRDY | 6 | RO | 1 | Indicate transmit ready status. 1: Transmit shift register is empty. 0: Transmit shift register is not empty. The SPI_WR_DATA_REG register is able to transmit data into transmit shift register only when this register field is set to empty. |
| #TMT | 5 | RO | 1 | Indicate transmit shift register is empty. 1: Transmit shift register is empty. 0: Transmit shift register is not empty. When the transmit shift register is empty, the next data will be transferred from SPI_WR_DATA_REG register. |
| #TOE | 4 | RO | 0 | Indicate transmit overrun error. 1: SPI_WR_DATA_REG register received new data before previous data was removed into the shift register. |
| #ROE | 3 | RO | 0 | Indicate receive overrun error. 1: SPI_RD_DATA_REG register received new data before previous data was read. 0: No new data received before previous data was read. |
| #RSVD | 2:0 | RO | 0 | Reserved |

## GEN_SPI_CTRL_REG

| #Register Name |  | GEN_SPI_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Instruct FPGA to select SPI peripheral and enable interrupt request. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:11 | RO | All 0s | Reserved. |
| #SSO | 10 | RW | 0 | Control the Chip Select output pin based on the mask provided in SPI_SLV_SEL_REG register. 1: Assert the Chip Select for the SPI slave device 0: De-assert the Chip Select for the SPI slave device. SPI Master will continue to exchange data frames with correspondent SPI slave when this register field is asserted. |
| #RSVD | 9:8 | RO | 0 | Reserved |
| #IRRDY | 7 | RW | 0 | Enable interrupt request for receiver ready. 1: Interrupt enabled. 0: Interrupt disabled. |
| #RSVD | 6:0 | RO | 0 | Reserved |

## GEN_SPI_CFG_CTRL_REG

| #Register Name |  | GEN_SPI_CFG_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Instruct FPGA to configure the SPI Master |  |  |
| POR Value |  | 0x0000_0210 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:15 | RO | All 0s | Reserved. |
| #SPI_MODE | 14:13 | RW | 00 | Control the SPI mode for the SPI transaction. 00 : Data sampled on rising edge and shifted out on falling edge (Clock polarity and clock phase is low.) 01 : Data sampled on the falling edge and shifted out on rising edge.(Clock polarity is low and clock phase is high) 10 : Data sampled on falling edge and shifted out on rising edge (Clock polarity is high and clock phase is low.) 11 : :Data sampled on rising edge and shifted out on falling edge (Clock polarity is high and clock phase is high.) |
| #SPI_SHIFT_BIT | 12 | RW | 0 | Control whether the MSB or LSB bit is shifted out first. 0: MSB bit is shifted out first 1: LSB bit is shifted out first |
| #SPI_CHP_SEL_DELAY | 11:8 | RW | 0x2 | Specify the delay based on half of the SPI clock period for the delay to send out the first data after Chip Select pin is asserted. 0x0: 0 SPI clock cycle 0x1: 0.5 SPI clock cycle 0x2: 1 SPI clock cycle …. 0xF: 7.5 SPI clock cycles |
| #RSVD | 7 | RO | 0 | Reserved. |
| #SPI_CLK_CNT | 6:4 | RW | 001 | The value in this field is the frequency divider of the system clock (75 MHz) to generate the serial clock for the SPI transaction. 000 75Mhz/2 (37.50 MHz) 001 75Mhz/4 (18.75 MHz) 010 75Mhz/6 (12.5 MHz) 011 75Mhz/8 (9.375 MHz) 100 75Mhz/10 (7.5 MHz) 101 75Mhz/12 (6.25 MHz) 110 75Mhz/14 (5.357 MHz) 111 75Mhz/16 (4.686 MHz) SPI clock frequency in MHz = 75MHz / (2 * (#SPI_CLK_CNT + 1) |
| #RSVD | 3:2 | RO |  | Reserved. |
| #SPI_DATA_WIDTH | 1:0 | RW | 00 | Specify number of bits send per each write or read transaction to the SPI slave. 00: 8 bits 01: 16 bits 10: 24 bits 11: 32 bits |

## GEN_SPI_SLV_SEL_REG

| #Register Name |  | GEN_SPI_SLV_SEL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Instruct SPI Master to select which SPI slave to send transaction. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #SPI_SLV_SEL | 31: 0 | RW | All 0s | Bit 0: Control the SPI slave 0 enable for SPI master access where 1 is enable and 0 is disable. … Bit 31: Control the SPI slave 0 enable for SPI master access where 1 is enable and 0 is disable. |

## MDIO_STAT_CMD_REG

| #Register Name |  | MDIO_STAT_CMD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | CPU Control of MDIO Master IP |  |  |
| POR Value |  | 0x8000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #MDIO_STATUS | 31 | RO | 1 | 0: MDIO transaction in progress 1: MDIO transaction completed It returns the status of the MDIO state machine. Once a CPU write to the MDIO register has taken place, the state machine begins to serialize the command to the transceiver. During this period, the status bit returns 0 every time the CPU reads from the register, and CPU writes will have no effect. As soon as the operation completes and the MDIO state machine has returned to idle, the status bit returns a one, and the CPU can safely read the data field or write a new command to the MDIO. This value is only cleared (1'b0) when a CPU MDIO transaction is pending or in progress. Otherwise it is set (1’b1) to represent that the last CPU MDIO transaction completed correctly. |
| #RSVD | 30 | RO | 0 | Reserved |
| #START | 29:28 | RW | 0 | Indicate the start of an MDIO transaction. These start bits differ between Clause 22 and Clause 45 MDIO. Note: Start bits are transmitted lsb first, so software must program them in reverse order as shown below. Clause 22 - start sequence 01, program start <= 10 Clause 45 - start sequence 00, program start <= 00 |
| #OPCODE | 27:26 | RW | All 0s | Determines the operation to be performed by the Management interface. The available opcodes differ between Clause 22 and Clause 45 MDIO. Clause 22 MDIO applies to 10/100/1000 Mbs MACs and Clause 45 MDIO applies to 10 GIG MACs. Note: Op-code bits are transmitted lsb first, so software must program them in reverse order as shown below. See long description for details. Clause 22 IEEE-opcode ASIC.opcode Description 00 00 - Not valid 01 10 - Write 10 01 - Read 11 11 - Not valid Clause 45 IEEE-opcode ASIC.opcode Description 00 00 Address 01 10 Write 11 11 Read 10 01 Post-read Increment Address |
| #PHYAD | 25:21 | RW | All 0s | It indicates which of 32 possible physical devices transceivers attached to this bus is being addressed. |
| #REGAD | 20:16 | RW | All 0s | It indicates which of 32 possible registers within the physical device the CPU wishes to read/write. |
| #DATA | 15:0 | RW | All 0s | The value in this field corresponds to the 16 bit value defined in the physical layer device data sheet or the 802.3u specification. On writes to the physical layer device, management would write the value to be sent to the physical layer device in this field while it was writing the opcode and address fields. On reads from the physical layer device, management will write the opcode and address fields. It will then monitor the status bit. When status bit is set (idle), it knows the data field has the value read from the physical layer device. Writes to this field are ignored when not idle. Reads of this field may contain invalid data when not idle. |

## MDIO_CTRL_REG

| #Register Name |  | MDIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | General control of MDIO Master IP |  |  |
| POR Value |  | 0x00F8_0007 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:24 | RO | All 0s | Reserved |
| #MDIO_PREAMBLE_CNT | 23:19 | RW | 1F | Programmable preamble for both CPU and MDIO Stack issued MDIO commands. NOTE: For proper operation with some Phys, this value should be set to 31. |
| #RSVD | 18:12 | RO | All 0s | Reserved |
| #STACK_EN | 11 | RW | 0 | 1: Enables the MDIO Stack. 0: Disable MDIO Stack. MDIO under CPU control with MDIO_STAT_CMD_REG. |
| #RSVD | 10:8 | RO | All 0s | Reserved |
| #MDIO_PROG_CLK_DIV | 7:0 | RW | 7 | This register sets the mdio clock divide rate. Period of clock = ((MDIO_PROG_CLK_DIV + 1)*13.33ns)*2 14 (8’hE) : 2.5 MHz 7 (8’h7) : 4.6875 MHZ 4 (8’h4) : 7.5 MHz 2 (8’h2) : 12.5 MHz 0 (8’h0) : 37.5 MHz |

## MDIO_STK_CTRL_REG

| #Register Name |  | MDIO_STK_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 | Reserved |
| #STACK_CMD_START | 0 | RW | 0 | 1: Enables MDIO stack operation 0: Legacy CPU-only operation using MDIO_STAT_CMD_REG. Note: Stack will stop executing after the current instruction if #STACK_EN is disabled before #STACK_END_ADD MDIO instruction is reached. #STACK_END_ADD is the address of #STACK_START_ADD + #STACK_NUM_CMDS |

## MDIO_STK_ADDR_CONF_REG

| #Register Name |  | MDIO_STK_ADDR_CONF_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0080 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:19 | RO | 0 | Reserved |
| #STACK_PROG_DELAY | 18:14 | RW | All 0s | The number of MDIO clock cycles (MDC) to way in between each MDIO stack command. (This is provided just in case some PHY has issues with quick back-to-back MDIO transactions.) |
| #STACK_NUM_CMDS | 13:7 | RW | 1 | The number of MDIO Stack RAM instructions/entries to execute starting at #STACK_START_ADDR. (Notes: Reading past the end of the stack is also and END condition) |
| #STACK_START_ADDR | 6:0 | RW | All 0s | Starting MDIO instruction in the stack to begin executing when #STACK_CMD_START and #STACK_EN are set |

## MDIO_STK_STAT_REG

| #Register Name |  | MDIO_STK_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0003 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | 0 | Reserved |
| #STACK_IDLE | 1 | RO | 1 | For DEBUG use only, should not use by SW Indicates that the MDIO stack has completed its instruction and there are no longer any MDIO commands to be executed. This field will be cleared whenever a new MDIO stack instruction has been issued with a non-zero number of MDIO commands. |
| #STACK_CMD_DONE | 0 | RO | 1 | 1: Stack processing done for all MDIO transactions of a given MDIO stack instruction specified in MDIO_STK_ADDR_CONF_REG and started from MDIO_STK_CTRL_REG. It also means the last #STACK_NUM_CMDS number of commands it was instructed to execute have been successfully completed. 0: This value is only cleared when the MDIO Stack has MDIO transactions pending or in progress |

## MDIO_STK_RAM_DATA_REG

| #Register Name |  | MDIO_STK_RAM_DATA_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | Individual stack value entry for corresponding #STACK_ADDR |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:30 | RO | All 0s | Reserved |
| #START | 29:28 | RW | All 0s | See MDIO_STAT_CMD_REG #START |
| #OPCODE | 27:26 | RW | All 0s | See MDIO_STAT_CMD_REG #OPCODE |
| #PHYAD | 25:21 | RW | All 0s | See MDIO_STAT_CMD_REG #PHYAD |
| #REGAD | 20:16 | RW | All 0s | See MDIO_STAT_CMD_REG #REGAD |
| #DATA | 15:0 | RW | All 0s | See MDIO_STAT_CMD_REG #DATA |

## MDIO_STK_RAM_ADDR_REG

| #Register Name |  | MDIO_STK_RAM_ADDR_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | MDIO Stack address |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 | Reserved |
| #STACK_ADDR | 7:0 | RW | 0 | Indicates address/position of stack that MDIO_STK_RAM_DATA_REG is writing/reading to |

## MDIO_STAT_INT_REG

| #Register Name |  | MDIO_STAT_INT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 2 |  |  |
| Description |  | MDIO Status interrupt |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:6 | RO | 0 | Reserved |
| #MDIO_STATUS_INT | 5 | W1C | 0 | Indicates a Single MDIO transaction is completed |
| #STK_CMD_DONE_INT | 4 | W1C | 0 | Indicates a new stack of MDIO transactions is done. |
| #RSVD | 3:2 | RO | 0 | Reserved |
| #MDIO_STATUS_INT_EN | 1 | RW | 0 | Enable Single MDIO interrupt (rising edge of MDIO_STAT_CMD_REG #MDIO_STATUS) |
| #STK_CMD_DONE_INT_EN | 0 | RW | 0 | Enable Stack MDIO interrupt (rising edge of MDIO_STK_STAT_REG #STK_CMD_DONE) |

## Function 3

## FPGA_REV_ID_REG

## FUNC_CAP_0_REG

## SCRTCH_n_REG

## DEV_MSI_0_PEND_REG (TBD)

| #Register Name |  | DEV_MSI_0_PEND_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 3 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #MBOX_INT | 0 | RO | 0 | 1: MBOX requires service. 0: no service is requested. |

## DEV_MSI_0_INT_EN_REG (TBD)

| #Register Name |  | DEV_MSI_0_INT_EN_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | CB |  |  |
| Function |  | 3 |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | 0 | Reserved |
| #MBOX_INT_EN | 0 | RW | 0 | 1: MBOX interrupt is enabled. 0: MBOX interrupt is disabled. |

## I2C_PRSCL_LO_REG

## I2C_PRSCL_HI_REG

## I2C_CTRL_REG

## I2C_TX_REG

## I2C_RX_REG

## I2C_CMD_REG

## I2C_STAT_REG

## I2C_MUX_SEL_REG

## I2C_RST_REG

# SWB Registers

## FPGA_REV_ID_REG

## FPGA_RECONFIG_CTRL_REG

## FPGA_RECONFIG_STAT_REG

## SCRTCH_n_REG

## SWB_INFO_REG

| #Register Name |  | SWB_INFO_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #TRAY_2_ID | 11:8 | RO | All 0s | Current Tray 2 ID value |
| #TRAY_1_ID | 7:4 | RO | All 0s | Current Tray 1 ID value |
| #RSVD | 3:1 | RO | All 0s | Reserved |
| #SWB_ID | 0 | RO | 0 | Current SWB ID 0: SWB0 1: SWB1 |

## PWR_STAT_REG

| #Register Name |  | PWR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #SWB_ALL_PGOK | 16 | RO | 0 | Indicates SWB power sequencing is completed 1: Asserted 0: De-asserted |
| #RSVD | 15:1 | RO | All 0s | Reserved |
| #TH6_VDD_CORE_VRM_INT | 0 | RO | 0 | TH6 VDD Power Interrupt 1: Asserted 0: De-asserted |

## CB_CTRL_REG

| #Register Name |  | CB_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:12 | RO | All 0s | Reserved |
| #LSB_RST_SWB | 11:8 | RW | 0 | Reset control of remote LSB 0xD: Asserted Any other value: de-asserted. When asserted by SW, will reset the SWB LSB port and all related processors. This Field is automatically cleared by FPGA after reset is issued. |
| #RSVD | 7:0 | RO | 0 | Reserved |

## CB_STAT_REG

| #Register Name |  | CB_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:28 | RO | All 0s | Reserved |
| #LSB_PROC_TYP_ERR | 27 | RO | 0 | One of the processors in LSB port received a packet with an invalid value in type field. |
| #LSB_PROC_FATAL_ERR | 26 | RO | 0 | One of the state machines in one of the processors in LSB port has gone momentarily to unknown state. |
| #LSB_PROC_EOP_ERR | 25 | RO | 0 | One of the processors in LSB port received a packet without EOP. |
| #LSB_PROC_SOP_ERR | 24 | RO | 0 | One of the processors in LSB port received a packet without SOP. |
| #RSVD | 23:19 | RO | 0 | Reserved |
| #LSB_TX_FATAL_ERR | 18 | RO | 0 | One of the state machines in LSB TX PHY has gone momentarily to unknown state. |
| #LSB_TX_EOP_ERR | 17 | RO | 0 | LSB TX PHY received a packet without EOP from one of the processors. |
| #LSB_TX_SOP_ERR | 16 | RO | 0 | LSB TX PHY received a packet without SOP from one of the processors. |
| #RSVD | 15 | RO | 0 | Reserved |
| #LSB_RX_REV_NO_ERR | 14 | RO | 0 | REV of the received packet does not match with the REV_NO of the LSB RX PHY. |
| #LSB_RX_UNEXP_ERR | 13 | RO | 0 | Unexpected condition in LSB RX PHY. 1: means RX_BUF is almost full or LSB_RX_PHY_CTRL has received a packet without SOP from DES (deserializer) |
| #LSB_RX_SEQNO_ERR | 12 | RO | 0 | LSB RX PHY received a packet from remote party with invalid SEQ no. |
| #LSB_RX_DEV_ID_ERR | 11 | RO | 0 | LSB RX PHY received a packet from remote party with invalid DEV-ID. |
| #LSB_RX_FATAL_ERR | 10 | RO | 0 | One of the state machines in LSB RX PHY has gone momentarily to unknown state. |
| #LSB_RX_LNGTH_ERR | 9 | RO | 0 | LSB RX PHY received a packet from remote party with invalid Length. |
| #LSB_RX_CHKSM_ERR | 8 | RO | 0 | LSB RX PHY received a packet from remote party with checksum error. |
| #RSVD | 7:0 | RO | 0 | Reserved |

## ASIC_CTRL_REG

| #Register Name |  | ASIC_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x00DD_03DD |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:24 | RO | All 0s | Reserved |
| #M0_CORE_RST | 23:20 | RW | 0xD | M0 Core Reset 0: Asserted 1: De-asserted |
| #R8_CORE_RST | 19:16 | RW | 0xD | R8 Core Reset 0: Asserted 1: De-asserted |
| #RSVD | 15:14 | RO | All 0s | Reserved |
| #TH6_PCIE_LANE | 13:12 | RW | 0x0 | Select the maximum link width of PCIe interface: 0: x1 / x2 / x4 link width 1: x1 link width 2: x1 or x2 link width 3: Reserved |
| #RSVD | 11:10 | RO | All 0s | Reserved |
| #TH6_PCIE_GEN | 9:8 | RW | 0x3 | Select the operating rate of PCIe interface: 0: Gen1 1: Gen2 2: Gen3 3: Gen4 |
| #TH6_PERST | 7:4 | RW | 0xD | TH6 PCIe Reset 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |
| #TH6_SYS_RST | 3:0 | RW | 0xD | TH6 System Reset 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |

## ASIC_STAT_REG

| #Register Name |  | ASIC_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #TH6_PHY_LINK_UP | 0 | RO | 0 | TH6 PHY Link status 1: Link is up 0: Link is down |

## RTM_CTRL_REG

| #Register Name |  | RTM_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_011D |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:9 | RO | All 0s | Reserved |
| #RTM_MODESEL | 8 | RW | 1 | Enable of Mode Select of all Retimer (1-56) 1: Shared SPI ROM 0: Single SPI ROM |
| #RSVD | 7:5 | RO | All 0s | Reserved |
| #RTM_SERBOOT | 4 | RW | 1 | Enable of Serial Boot of all Retimers (1-56) 1: from SPI ROM 0: from MDIO |
| #RTM_RST | 3:0 | RW | 0xD | Reset of all Retimers (1-56) 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |

## THRM_STAT_REG

| #Register Name |  | THRM_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #TH6_TS_HW_OTP_INT | 1 | RO | 0 | TH6 thermal diodes temperature sensor HW OTP shutdown interrupt 1: Asserted 0: De-asserted |
| #EXT_TEMPSNSR_ALERT | 0 | RO | 0 | Multiple external temperature sensor alert 1: Asserted 0: De-asserted |

## EN_ACCSS_PRTCTD_REG

This register is to protect critical registers from accidental writes. SW should write 0x8F35_D72C in order to have write access to protected registers.

| #Register Name |  | EN_ACCSS_PRTCTD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | PMC |  |  |
| Function |  | None |  |  |
| Description |  | Enables access to protected registers. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #EN_ACCSS | 31:0 | RW | 0 | 0x8F35_D72C: Enables access to protected registers. Other Values: Disables access to protected registers. |

## I2C_MUX_RST_REG

| #Register Name |  | I2C_MUX_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #I2C_MUX1_RST | 7:4 | RW | 0x0 | Reset control of I2C MUX which has I2Cs of CB_FPGA to SWB external temperature sensors. 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |
| #I2C_MUX0_RST | 3:0 | RW | 0x0 | Reset control of I2C MUX which has I2Cs of CB_FPGA to clock gen, EEPROM and tray cartridge 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |

## SPI_MUX_REG

| #Register Name |  | SPI_MUX_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:9 | RO | All 0s | Reserved |
| #TH6_SPI_MUX_SEL | 8 | RW | 0 | MUX Select for TH6 SPI flash connection from: 1: SWB FPGA 0: TH6 ASIC |
| #RSVD | 7:4 | RO | All 0s | Reserved |
| #SWB_SPI_MUX_SEL | 4 | RW | 0 | MUX Select for SWB SPI flash connection from: 1: CB FPGA 0: SWB FPGA |
| #RSVD | 3:1 | RO | All 0s | Reserved |
| #CB_SPI_MUX_SEL | 0 | RW | 0 | MUX Select for CB SPI connection to: 1: Direct access to SWB SPI Flash 0: Pass-through SWB_FPGA (to access TH6 SPI flash) |

## FLSH_CTRL_REG

This register is clear under two conditions only.

- Upon assertion of global reset.

- Power cycle

| #Register Name |  | FLSH_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:13 | RO | All 0s | Reserved |
| #RTM_FLSH_WP | 12 | RW | 0 | Retimers Flash Write Protect 1: Protected, Once asserted, FPGA will not issue any write/erase command to the flash. Once asserted, cannot be de-asserted by SW. |
| #RSVD | 11:9 | RO | All 0s | Reserved |
| #TH6_FLSH_WP | 8 | RW | 0 | TH6 Flash Write Protect 1: Protected, Once asserted, FPGA will not issue any write/erase command to the flash. Once asserted, cannot be de-asserted by SW. |
| #RSVD | 7:5 | RO | All 0s | Reserved |
| #DIAG_EEPRM_WP | 4 | RW | 0 | Diagnostic EEPROM Write Protect 1: Protected, Once asserted, FPGA will not issue any write/erase command to the flash. Once asserted, cannot be de-asserted by SW. |
| #RSVD | 3:2 | RO | All 0s | Reserved |
| #FPGA_FLSH_WP | 1 | RW | 0 | FPGA (CB) Flash Write Protect 1: Protected, Once asserted, FPGA will not issue any write/erase command to the flash. Once asserted, cannot be de-asserted by SW. |
| #RSVD | 0 | RO | 0 | Reserved |

x

## TND_PWR_CTRL_0_REG

This is a protected register by EN_ACCSS_PRTCTD_REG.

| #Register Name |  | TND_PWR_CTRL_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x001F_FFFF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:30 | RO | All 0s | Reserved |
| #DIPSW_DEBUGBOOT | 29 | RW | 0 | Debug Boot value |
| #JMP_3ASTG_BOOT | 28 | RW | 0 | 3AStg Boot value |
| #RSVD | 27:25 | RO | All 0s | Reserved |
| #PSSM_OVR | 24 | RW | 0 | Enable override of PSSM 1: Enable 0: Disable |
| #RSVD | 23:21 | RO | All 0s | Reserved |
| #PWREN_PVDD1V2_1 | 20 | RW | 1 |  |
| #PWREN_PVDD1V2_0 | 19 | RW | 1 |  |
| #PWREN_PVDD1V8_DUT | 18 | RW | 1 |  |
| #PWREN_PVDD_MDIO | 17 | RW | 1 |  |
| #PWREN_P1V8_DVDDIO_4 | 16 | RW | 1 |  |
| #PWREN_P1V8_DVDDIO_3 | 15 | RW | 1 |  |
| #PWREN_P1V8_DVDDIO_2 | 14 | RW | 1 |  |
| #PWREN_P1V8_DVDDIO_1 | 13 | RW | 1 |  |
| #PWREN_P0V75_DVDD_57 | 12 | RW | 1 |  |
| #PWREN_P0V75_DVDD_50 | 11 | RW | 1 |  |
| #PWREN_P0V75_DVDD_43 | 10 | RW | 1 |  |
| #PWREN_P0V75_DVDD_36 | 9 | RW | 1 |  |
| #PWREN_P0V75_DVDD_21 | 8 | RW | 1 |  |
| #PWREN_P0V75_DVDD_14 | 7 | RW | 1 |  |
| #PWREN_P0V75_DVDD_7 | 6 | RW | 1 |  |
| #PWREN_P0V75_DVDD_0 | 5 | RW | 1 |  |
| #PWREN_P1V8 | 4 |  |  |  |
| #PWREN_PVDD_3V_SYNTH_LDO_B1 | 3 | RW | 1 |  |
| #PWREN_PVDD_3V_SYNTH_LDO_A1 | 2 | RW | 1 |  |
| #PWREN_P3V3 | 1 | RW | 1 |  |
| #PWREN_P5V_STBY | 0 | RW | 1 |  |

## TND_PWR_CTRL_1_REG

This is a protected register by EN_ACCSS_PRTCTD_REG.

| #Register Name |  | TND_PWR_CTRL_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x3FFF_FFFF |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:30 | RO | All 0s | Reserved |
| #CLKEN_BOT_TOP_PLL_CLKIN | 29 | RW | 1 |  |
| #PWREN_PVDD1V2_T3 | 28 | RW | 1 |  |
| #PWREN_PVDD1V45_3 | 27 | RW | 1 |  |
| #PWREN_PVDD1V45_2 | 26 | RW | 1 |  |
| #PWREN_PVDD1V45_1 | 25 | RW | 1 |  |
| #PWREN_PVDD1V45_0 | 24 | RW | 1 |  |
| #PWREN_P1V5_CNDR1 | 23 | RW | 1 |  |
| #PWREN_P1V5_CNDR0 | 22 | RW | 1 |  |
| #PWREN_PVDD1V5_ANLG | 21 | RW | 1 |  |
| #PWREN_PVDD_0V9_3 | 20 | RW | 1 |  |
| #PWREN_PVDD_0V9_2 | 19 | RW | 1 |  |
| #PWREN_PVDD_0V9_1 | 18 | RW | 1 |  |
| #PWREN_PVDD_0V9_0 | 17 | RW | 1 |  |
| #PWREN_PVDD_0V9_ANLG_1 | 16 | RW | 1 |  |
| #PWREN_PVDD_0V9_ANLG_0 | 15 | RW | 1 |  |
| #PWREN_PVDD0_8V_T3 | 14 | RW | 1 |  |
| #PWREN_PVDDA_PHYT3_P3 | 13 | RW | 1 |  |
| #PWREN_PVDDA_PHYT2_P2 | 12 | RW | 1 |  |
| #PWREN_PVDDA_PHYT1_P1 | 11 | RW | 1 |  |
| #PWREN_PVDD0V75_ANLG_1 | 10 | RW | 1 |  |
| #PWREN_PVDD0V75_ANLG_0 | 9 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_7 | 8 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_6 | 7 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_5 | 6 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_4 | 5 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_3 | 4 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_2 | 3 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_1 | 2 | RW | 1 |  |
| #PWREN_PVDD_PHYTILE_0 | 1 | RW | 1 |  |
| #PWREN_FM_PVDD_CORE | 0 | RW | 1 |  |

## TND_PWR_STAT_0_REG

| #Register Name |  | TND_PWR_STAT_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #PG_P0V75_DVDD_26 | 31 |  |  |  |
| #PG_P0V75_DVDD_25 | 30 |  |  |  |
| #PG_P0V75_DVDD_24 | 29 |  |  |  |
| #PG_P0V75_DVDD_23 | 28 |  |  |  |
| #PG_P0V75_DVDD_22 | 27 |  |  |  |
| #PG_P0V75_DVDD_21 | 26 |  |  |  |
| #PG_P0V75_DVDD_20 | 25 |  |  |  |
| #PG_P0V75_DVDD_19 | 24 |  |  |  |
| #PG_P0V75_DVDD_18 | 23 |  |  |  |
| #PG_P0V75_DVDD_17 | 22 |  |  |  |
| #PG_P0V75_DVDD_16 | 21 |  |  |  |
| #PG_P0V75_DVDD_15 | 20 |  |  |  |
| #PG_P0V75_DVDD_14 | 19 |  |  |  |
| #PG_P0V75_DVDD_13 | 18 |  |  |  |
| #PG_P0V75_DVDD_12 | 17 |  |  |  |
| #PG_P0V75_DVDD_11 | 16 |  |  |  |
| #PG_P0V75_DVDD_10 | 15 |  |  |  |
| #PG_P0V75_DVDD_9 | 14 |  |  |  |
| #PG_P0V75_DVDD_8 | 13 |  |  |  |
| #PG_P0V75_DVDD_7 | 12 |  |  |  |
| #PG_P0V75_DVDD_6 | 11 |  |  |  |
| #PG_P0V75_DVDD_5 | 10 |  |  |  |
| #PG_P0V75_DVDD_4 | 9 |  |  |  |
| #PG_P0V75_DVDD_3 | 8 |  |  |  |
| #PG_P0V75_DVDD_2 | 7 |  |  |  |
| #PG_P0V75_DVDD_1 | 6 |  |  |  |
| #PG_P0V75_DVDD_0 | 5 |  |  |  |
| #PG_P1V8 | 4 |  |  |  |
| #PG_PVDD_3V_SYNTH_LDO_B1 | 3 |  |  |  |
| #PG_PVDD_3V_SYNTH_LDO_A1 | 2 |  |  |  |
| #PG_P3V3 | 1 |  |  |  |
| #PG_P5V_STBY | 0 |  |  |  |

## TND_PWR_STAT_1_REG

| #Register Name |  | TND_PWR_STAT_1_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #PG_P1V8_DVDDIO_3 | 31 |  |  |  |
| #PG_P1V8_DVDDIO_2 | 30 |  |  |  |
| #PG_P1V8_DVDDIO_1 | 29 |  |  |  |
| #PG_P0V75_DVDD_63 | 28 |  |  |  |
| #PG_P0V75_DVDD_62 | 27 |  |  |  |
| #PG_P0V75_DVDD_61 | 26 |  |  |  |
| #PG_P0V75_DVDD_60 | 25 |  |  |  |
| #PG_P0V75_DVDD_59 | 24 |  |  |  |
| #PG_P0V75_DVDD_58 | 23 |  |  |  |
| #PG_P0V75_DVDD_57 | 22 |  |  |  |
| #PG_P0V75_DVDD_56 | 21 |  |  |  |
| #PG_P0V75_DVDD_55 | 20 |  |  |  |
| #PG_P0V75_DVDD_54 | 19 |  |  |  |
| #PG_P0V75_DVDD_53 | 18 |  |  |  |
| #PG_P0V75_DVDD_52 | 17 |  |  |  |
| #PG_P0V75_DVDD_51 | 16 |  |  |  |
| #PG_P0V75_DVDD_50 | 15 |  |  |  |
| #PG_P0V75_DVDD_49 | 14 |  |  |  |
| #PG_P0V75_DVDD_48 | 13 |  |  |  |
| #PG_P0V75_DVDD_47 | 12 |  |  |  |
| #PG_P0V75_DVDD_46 | 11 |  |  |  |
| #PG_P0V75_DVDD_45 | 10 |  |  |  |
| #PG_P0V75_DVDD_44 | 9 |  |  |  |
| #PG_P0V75_DVDD_43 | 8 |  |  |  |
| #PG_P0V75_DVDD_42 | 7 |  |  |  |
| #PG_P0V75_DVDD_41 | 6 |  |  |  |
| #PG_P0V75_DVDD_40 | 5 |  |  |  |
| #PG_P0V75_DVDD_39 | 4 |  |  |  |
| #PG_P0V75_DVDD_38 | 3 |  |  |  |
| #PG_P0V75_DVDD_37 | 2 |  |  |  |
| #PG_P0V75_DVDD_36 | 1 |  |  |  |
| #PG_P0V75_DVDD_27 | 0 |  |  |  |

## TND_PWR_STAT_2_REG

| #Register Name |  | TND_PWR_STAT_2_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #PG_PVDD1V45_2 | 31 |  |  |  |
| #PG_PVDD1V45_1 | 30 |  |  |  |
| #PG_PVDD1V45_0 | 29 |  |  |  |
| #PG_P1V5_CNDR1 | 28 |  |  |  |
| #PG_P1V5_CNDR0 | 27 |  |  |  |
| #PG_PVDD1V5_ANLG | 26 |  |  |  |
| #PG_PVDD_0V9_3 | 25 |  |  |  |
| #PG_PVDD_0V9_2 | 24 |  |  |  |
| #PG_PVDD_0V9_1 | 23 |  |  |  |
| #PG_PVDD_0V9_0 | 22 |  |  |  |
| #PG_PVDD_0V9_ANLG_1 | 21 |  |  |  |
| #PG_PVDD_0V9_ANLG_0 | 20 |  |  |  |
| #PG_PVDD0_8V_T3 | 19 |  |  |  |
| #PG_PVDDA_PHYT3_P3 | 18 |  |  |  |
| #PG_PVDDA_PHYT2_P2 | 17 |  |  |  |
| #PG_PVDDA_PHYT1_P1 | 16 |  |  |  |
| #PG_PVDD0V75_ANLG_1 | 15 |  |  |  |
| #PG_PVDD0V75_ANLG_0 | 14 |  |  |  |
| #PG_PVDD_PHYTILE_7 | 13 |  |  |  |
| #PG_PVDD_PHYTILE_6 | 12 |  |  |  |
| #PG_PVDD_PHYTILE_5 | 11 |  |  |  |
| #PG_PVDD_PHYTILE_4 | 10 |  |  |  |
| #PG_PVDD_PHYTILE_3 | 9 |  |  |  |
| #PG_PVDD_PHYTILE_2 | 8 |  |  |  |
| #PG_PVDD_PHYTILE_1 | 7 |  |  |  |
| #PG_PVDD_PHYTILE_0 | 6 |  |  |  |
| #PG_FM_PVDD_CORE | 5 |  |  |  |
| #PG_PVDD1V2_1 | 4 |  |  |  |
| #PG_PVDD1V2_0 | 3 |  |  |  |
| #PG_PVDD1V8_DUT | 2 |  |  |  |
| #PG_PVDD_MDIO | 1 |  |  |  |
| #PG_P1V8_DVDDIO_4 | 0 |  |  |  |

## TND_PWR_STAT_3_REG

| #Register Name |  | TND_PWR_STAT_3_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #PG_PVDD1V2_T3 | 1 |  |  |  |
| #PG_PVDD1V45_3 | 0 |  |  |  |

## TND_CB_GPIO_CTRL_REG

| #Register Name |  | TND_CB_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 | Reserved |
| #GPI_7 | 7 | RW | 0 |  |
| #GPI_6 | 6 | RW | 0 | 1: Assert GPIO 6 0: De-assert GPIO 6 |
| #RSVD | 5:0 | RO | 0 | Reserved |

## TND_CB_GPIO_STAT_REG

| #Register Name |  | TND_CB_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #GPO_7 | 7 | RO | 0 |  |
| #GPO_6 | 6 | RO | 0 | 1: GPIO 6 is asserted 0: GPIO 6 is de-asserted |
| #RSVD | 5:0 | RO | All 0s | Reserved |

## TND_CB_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_CB_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | 0 | Reserved |
| #GPIO_DIR_7 | 7 | RW | 0 |  |
| #GPIO_DIR_6 | 6 | RW | 0 | 1: Indicate GPIO 6 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 6 is configured as input and register GPIO_STAT_REG store this input port value. |
| #RSVD | 5:0 | RO | 0 | Reserved |

## RST_CAUSE_REG

This is a protected register. This is sticky and does not get reset under global reset condition.

| #Register Name |  | RST_CAUSE_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0001 |  |  |
| Field | Bits | Type | Reset | Description |
| #SW_RST_COOKIES | 31:16 | RW | All 0s | For SW to read and write. |
| #RSVD | 11:9 | RO | All 0s | Reserved. |
| #CLD_RST | 8 | W1C | 0 | This bit will be set if the System PGOOD is de-sserted from a good state. |
| #RSVD | 7:1 | RO | All 0s | Reserved. |
| #PWR_ON_RST | 0 | W1C | 1 |  |

# ICB Registers

## FPGA_REV_ID_REG

## FPGA_RECONFIG_CTRL_REG

## FPGA_RECONFIG_STAT_REG

## SCRTCH_n_REG

## PWR_CTRL_REG

| #Register Name |  | PWR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0099 |  |  |
| Field | Bits | Type | Reset | Description |
| #PWR_SUICIDE | 31:16 | RW | All 0s | Value of 0xA73D in this register will disable 1Khz signal sent out to the hardware external watchdog and toggle the reconfiguration pin of ICB. This field is used for ICB post-ISP update. |
| #RSVD | 15:8 | RO | All 0s | Reserved. |
| #P12V_VRM_EN_SWB_1 | 7:4 | RW | 0x9 | SWB1 VRM enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |
| #P12V_VRM_EN_SWB_0 | 3:0 | RW | 0x9 | SWB0 VRM enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. |

## PWR_STAT_REG

| #Register Name |  | PWR_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_00F7 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:21 | RO | All 0s | Reserved |
| #ALRT_P50V_FAN_8 | 20 | RO | 0 |  |
| #ALRT_P50V_FAN_7 | 19 | RO | 0 |  |
| #ALRT_P50V_FAN_6 | 18 | RO | 0 |  |
| #ALRT_P50V_FAN_5 | 17 | RO | 0 |  |
| #ALRT_P50V_FAN_4 | 16 | RO | 0 | Alert from Fan 4 1: Asserted 0: De-asserted |
| #RSVD | 15:13 | RO | All 0s | Reserved |
| #PG_P50V_FAN_8 | 12 | RO | 0 |  |
| #PG_P50V_FAN_7 | 11 | RO | 0 |  |
| #PG_P50V_FAN_6 | 10 | RO | 0 |  |
| #PG_P50V_FAN_5 | 9 | RO | 0 |  |
| #PG_P50V_FAN_4 | 8 | RO | 0 | Power status of Fan 4 1: Power is ok |
| #ALRT_P50V_HSC_STBY | 7 | RO | 1 | Alert from hotswap controller standby 1: Asserted 0: De-asserted |
| #ALRT_P50V_HSC_MAIN | 6 | RO | 1 | Alert from hotswap controller main 1: Asserted 0: De-asserted |
| #PRSNT_P50V_HSC_STBY | 5 | RO | 1 | Present status of hotswap controller standby 1: Present 0: Not present |
| #PRSNT_P50V_HSC_MAIN | 4 | RO | 1 | Present status hotswap controller main 1: Present 0: Not present |
| #RSVD | 3 | RO | 0 | Reserved |
| #PG_P50V_HSC_STBY | 2 | RO | 1 | Power status of hotswap controller standby 1: Power is ok |
| #PG_P50V_HSC_MAIN | 1 | RO | 1 | Power status of hotswap controller main 1: Power is ok |
| #PG_P3V3_STBY | 0 | RO | 1 | Power status of P3V3 Standby power rail 1: Power is ok |

## FAN_CTRL_REG

| #Register Name |  | FAN_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0006_6666 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:20 | RO | All 0s | Reserved. |
| #P50V_EFUSE_EN_FAN_8 | 19:16 | RW | 0x6 | Fan 8 EFUSE enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. (Exception: this signal will automatically clear when fan is not present) |
| #P50V_EFUSE_EN_FAN_7 | 15:12 | RW | 0x6 | Fan 7 EFUSE enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. (Exception: this signal will automatically clear when fan is not present) |
| #P50V_EFUSE_EN_FAN_6 | 11:8 | RW | 0x6 | Fan 6 EFUSE enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. (Exception: this signal will automatically clear when fan is not present) |
| #P50V_EFUSE_EN_FAN_5 | 7:4 | RW | 0x6 | Fan 5 EFUSE enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. (Exception: this signal will automatically clear when fan is not present) |
| #P50V_EFUSE_EN_FAN_4 | 3:0 | RW | 0x6 | Fan 4 EFUSE enable 0x6: Enable 0x9: Disable Although any value can be written in this field by SW, but FPGA will not take action if the value is not 0x6 or 0x9 and will keep the previous Enable/Disable status. (Exception: this signal will automatically clear when fan is not present) |

## FAN_STAT_REG

| #Register Name |  | FAN_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:13 | RO | All 0s | Reserved |
| #FAN_FULL_SPD | 12 | RO | 0 | Fan speed status 1: Fan is at full speed 0: Fan is not at full speed (buffered signal from CB FPGA) |
| #RSVD | 11:9 | RO | All 0s | Reserved |
| #FAN_CTRLR_INT | 8 | RO | 0 | Interrupt status of fan controller (fan 4-8) 1: Asserted 0: De-asserted |
| #RSVD | 7:5 | RO | All 0s | Reserved |
| #PRSNT_FAN_8 | 4 | RO | 0 |  |
| #PRSNT_FAN_7 | 3 | RO | 0 |  |
| #PRSNT_FAN_6 | 2 | RO | 0 |  |
| #PRSNT_FAN_5 | 1 | RO | 0 |  |
| #PRSNT_FAN_4 | 0 | RO | 0 | Fan 4 presence status 1: Present 0: Not present |

## THRM_STAT_REG

| #Register Name |  | THRM_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:2 | RO | All 0s | Reserved |
| #TMP75_ALRT | 1 | RW | 0 | Interrupt from temperature sensor TMP75 1: Asserted 0: De-asserted |
| #TMP432_ALRT | 0 | RW | 0 | Interrupt from temperature sensor TMP432 1: Asserted 0: De-asserted |

## EN_ACCSS_PRTCTD_REG

This register is to protect critical registers from accidental writes. SW should write 0x8F35_D72C in order to have write access to protected registers.

| #Register Name |  | EN_ACCSS_PRTCTD_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  | Enables access to protected registers. |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #EN_ACCSS | 31:0 | RW | 0 | 0x8F35_D72C: Enables access to protected registers. Other Values: Disables access to protected registers. |

## I2C_MUX_RST_REG

| #Register Name |  | I2C_MUX_RST_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #I2C_MUX_RST | 3:0 | RW | 0x0 | Reset control of I2C MUX Channel 4 – VRM and Fan EFUSE. 0xD: Asserted Any other value: de-asserted. This Field is NOT automatically cleared by FPGA after reset is issued. |

## FLSH_CTRL_REG

This register is clear under two conditions only.

- Upon assertion of global reset.

- Power cycle

| #Register Name |  | FLSH_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:1 | RO | All 0s | Reserved |
| #FRU_EEPRM_WP | 0 | RW | 0 | FRU EEPROM Write Protect 1: Protected, Once asserted, cannot be de-asserted by SW. |

## LED_SYNC_REG

## LED_CONFIG_REG

## LED_CTRL_REG

For bit definitions please refer to the section “LED Control bits”.

| #Register Name |  | LED_CTRL_0_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | none |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:8 | RO | All 0s | Reserved |
| #FAN_ALRT_STAT | 7:0 | RW | 0x0 | Fan Alert LED status |

## TND_CB_GPIO_CTRL_REG

| #Register Name |  | TND_CB_GPIO_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #GPI_3 | 3 | RW | 0 |  |
| #GPI_2 | 2 | RW | 0 |  |
| #GPI_1 | 1 | RW | 0 | 1: Assert GPIO 1 0: De-assert GPIO 1 |
| #RSVD | 0 | RO | 0 | Reserved |

## TND_CB_GPIO_STAT_REG

| #Register Name |  | TND_CB_GPIO_STAT_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | SWB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | All 0s | Reserved |
| #GPO_3 | 3 | RO | 0 |  |
| #GPO_2 | 2 | RO | 0 |  |
| #GPO_1 | 1 | RO | 0 | 1: GPIO 1 is asserted 0: GPIO 1 is de-asserted |
| #RSVD | 0 | RO | 0 | Reserved |

## TND_CB_GPIO_DIR_CTRL_REG

| #Register Name |  | TND_CB_GPIO_DIR_CTRL_REG |  |  |
| --- | --- | --- | --- | --- |
| Card Type |  | All |  |  |
| FPGA Type |  | ICB |  |  |
| Function |  | None |  |  |
| Description |  |  |  |  |
| POR Value |  | 0x0000_0000 |  |  |
| Field | Bits | Type | Reset | Description |
| #RSVD | 31:4 | RO | 0 | Reserved |
| #GPIO_DIR_3 | 3 | RW | 0 |  |
| #GPIO_DIR_2 | 2 | RW | 0 |  |
| #GPIO_DIR_1 | 1 | RW | 0 | 1: Indicate GPIO 1 is configured as output and register GPIO_CTRL_REG is used to control this output port value. 0: Indicate GPIO 1 is configured as input and register GPIO_STAT_REG store this input port value. |
| #RSVD | 0 | RO | 0 | Reserved |